SIMATS ENGINEERING

Department of Computer Science & Engineering ASSESSMENT TOOL 1-ANALYTICAL PROBLEM SOLVING

| Course Code: | CSA12 | Course Title: | Computer Architecture |
| --- | --- | --- | --- |
| CO Assessed: | CO2 | Assessment: | ASSESSMENT TOOL1- ANALYTICAL PROBLEM SOLVING |
| Weightage: | 45% | Total Marks: | 25 |
| CO2Statement: | Apply integer and floating-point arithmetic algorithms including Booth's multiplication, restoring/non-restoring division, and IEEE 754 standard operations to solve fixed-point and floating-point computational problems. (BL3) |  |  |
| Student Name: |  | Reg.No.: |  |
| Department: |  | Date: |  |

ANNEXURE A

1. A processor performs arithmetic operations on signed 8-bit integers using 2’s complement  representation. Consider two numbers: +45 and −23. Analyze in detail how the system  performs both addition and subtraction, including binary conversion, alignment of sign bits,  and intermediate steps. Determine whether overflow occurs, and explain how the processor  detects overflow and interprets the final result.

2. A high-performance computing system replaces a ripple carry adder with a 4-bit carry look ahead adder (CLA) to reduce computation delay. Given two binary inputs, analyze how  generate and propagate signals are formed and how carries are computed in advance. Compare the time delay of CLA with ripple carry addition and justify, with reasoning, why  CLA improves speed in modern processors.

3. In a signed multiplication operation, a processor uses Booth’s algorithm to multiply two  numbers, −6 and +5. Analyze the algorithm step-by-step, including the role of the  accumulator, multiplier, and Booth bit, along with arithmetic shifts. Explain how the  algorithm efficiently handles signed numbers and reduces the number of addition/subtraction  operations compared to traditional multiplication.

4. A processor is required to perform division of two binary numbers (e.g., 13 ÷ 3) using both  restoring and non-restoring division algorithms. Analyze both methods step-by-step, showing  intermediate remainders and quotient formation. Compare the two approaches in terms of  number of steps, complexity, and efficiency, and explain why non-restoring division is often  preferred in modern systems.

5. A scientific application requires precise handling of real numbers using the IEEE 754  standard for floating-point representation. Given a decimal number (e.g., −13.25), analyze  how it is represented in both single precision and double precision formats, including sign,  exponent, and mantissa fields. Further, explain how floating-point addition is performed  between two such numbers, highlighting issues like normalization, rounding, and precision  loss.

Code of Conduct:

I DEEPTHI KORRAPATI (192572012) (Name / Reg No) certify that this  submission is my original work and that I have adhered to the guidelines specified for this  assessment.I understand that any violation of academic integrity rules will result in  disciplinary action.

Signature of the student:

MARKS ALLOCATION

|  | Problem Understanding(2 0%) | Method Selection(20%) | Solution Process(25%) | Accuracy of Calculation(20%) | Interpretation of Results(15%) |
| --- | --- | --- | --- | --- | --- |
| Q1 |  |  |  |  |  |
| Q2 |  |  |  |  |  |
| Q3 |  |  |  |  |  |
| Q4 |  |  |  |  |  |
| Q5 |  |  |  |  |  |

RUBRICS FOR CALCULATION:

| Criteria | Weightage | Level 4 – Exemplary | Level 3 – Proficient | Level 2 – Developing | Level 1 – Beginning |
| --- | --- | --- | --- | --- | --- |
| Problem Understanding | 20% | Clearly interprets problem, identifies all parameters and requirements accurately | Understands problem with minor gaps | Partial understanding; misses key elements | Misinterprets or unable to understand problem |
| Method Selection | 20% | Selects most appropriate method/formula with strong justification | Selects correct method with minor errors | Inappropriate or partially correct method | Unable to select method |
| Solution Process | 25% | Logical, systematic steps with correct approach throughout | Mostly correct steps with minor mistakes | Disorganized steps; multiple errors | No clear process |
| Accuracy of Calculation | 20% | All calculations accurate; correct final answer | Minor calculation errors | Frequent errors; incorrect final answer | No valid calculations |
| Interpretation of Results | 15% | Clearly interprets results with units and engineering relevance | Basic interpretation with minor omissions | Limited or unclear interpretation | No interpretation |

1.Perform the following operations using signed 8-bit 2's complement arithmetic. Show all binary calculations and determine whether overflow occurs.

a) +45 + (−23)

b) +45 − (−23)

solution:

Introduction

In digital computers, signed integers are represented using the 2's complement number system. This representation is widely used because it simplifies arithmetic operations, allowing both addition and subtraction to be performed using the same binary addition circuit. It also provides a unique representation for zero and simplifies overflow detection.

For an 8-bit signed number, the range is:

- Minimum value = −128

- Maximum value = +127

Since +45 and −23 fall within this range, they can be represented using 8-bit binary numbers.

Theory of 2's Complement

The 2's complement of a binary number is obtained in two steps:

- Find the 1's complement (invert all bits: 0 becomes 1 and 1 becomes 0).

- Add 1 to the least significant bit (LSB).

Example:

+23 = 00010111

1's Complement = 11101000

2's Complement = 11101001

Therefore,

−23 = 11101001

Part (a): Addition of +45 and (−23)

Step 1: Convert +45 into Binary

Divide 45 repeatedly by 2:

| Division | Quotient | Remainder |
| --- | --- | --- |
| 45 ÷ 2 | 22 | 1 |
| 22 ÷ 2 | 11 | 0 |
| 11 ÷ 2 | 5 | 1 |
| 5 ÷ 2 | 2 | 1 |
| 2 ÷ 2 | 1 | 0 |
| 1 ÷ 2 | 0 | 1 |

Reading the remainders from bottom to top:

45 = 101101₂

Represent it as an 8-bit number:

+45 = 00101101

Step 2: Convert −23 into Binary

Positive 23:

23 = 00010111

Take the 1's complement:

11101000

Add 1:

11101000

+00000001

----------

11101001

Hence,

−23 = 11101001

Step 3: Perform Binary Addition

Carry: 1110001

00101101   (+45)

+ 11101001   (−23)

-------------------

00010110

The carry out of the most significant bit is discarded.

Step 4: Convert the Result to Decimal

Binary result:

00010110

Decimal value:

16 + 4 + 2 = 22

Therefore,

+45 + (−23) = +22

Step 5: Overflow Detection

Overflow occurs only when:

- Positive + Positive = Negative

- Negative + Negative = Positive

In this problem:

Positive + Negative

Since the operands have different signs, overflow cannot occur.

Overflow = No

Part (b): Subtraction of +45 − (−23)

Subtracting a negative number is equivalent to adding the positive number.

45 − (−23)

= 45 + 23

Step 1: Binary Representation

+45 = 00101101

+23 = 00010111

Step 2: Binary Addition

Carry: 0011111

00101101

+ 00010111

----------------

01000100

Step 3: Convert Back to Decimal

Binary result:

01000100

Decimal value:

64 + 4 = 68

Hence,

+45 − (−23)

= +68

Step 4: Overflow Detection

The operands are positive.

The result is also positive.

Maximum value in an 8-bit signed number is 127.

Since

68 < 127

there is no overflow.

Final Results

| Operation | Binary Result | Decimal Result | Overflow |
| --- | --- | --- | --- |
| +45 + (−23) | 00010110 | +22 | No |
| +45 − (−23) | 01000100 | +68 | No |

Advantages of 2's Complement

- Simple representation of positive and negative numbers.

- Only one representation for zero.

- Addition and subtraction use the same hardware.

- Easy overflow detection.

- Faster arithmetic operations in processors.

- Most widely used number representation in modern computers.

Applications

- Arithmetic Logic Unit (ALU)

- Microprocessors

- Digital Signal Processing (DSP)

- Embedded Systems

- Computer Architecture

- Computer Arithmetic Operations

- Scientific Computing

Conclusion

The 2's complement representation is the standard method used by computers for performing signed arithmetic operations. It simplifies subtraction by converting it into addition and allows efficient implementation using digital circuits. In this problem:

- +45 + (−23) = +22 (No Overflow)

- +45 − (−23) = +68 (No Overflow)

Both results are within the valid range of an 8-bit signed integer (−128 to +127), so no overflow occurs. This method is widely used in modern processors because of its simplicity, speed, and efficient hardware implementation.

Question 2

Explain the working of a 4-bit Carry Look-Ahead Adder (CLA) with a suitable diagram. Compare it with a Ripple Carry Adder (RCA).

Answer

Introduction

Addition is one of the most frequently performed arithmetic operations in a computer. The speed of addition directly affects the performance of the CPU. A Carry Look-Ahead Adder (CLA) is a high-speed adder designed to reduce the delay caused by carry propagation in a Ripple Carry Adder (RCA).

Unlike the Ripple Carry Adder, where each full adder must wait for the carry from the previous stage, the Carry Look-Ahead Adder calculates all carry bits simultaneously using Generate (G) and Propagate (P) signals. This significantly improves the speed of binary addition.

Need for Carry Look-Ahead Adder

In a Ripple Carry Adder:

- Each carry must travel through every full adder.

- Delay increases with the number of bits.

- Performance decreases for large binary numbers.

To overcome these problems, the Carry Look-Ahead Adder predicts carry bits in advance instead of waiting for them to propagate.

Block Diagram of a 4-bit Carry Look-Ahead Adder

A3  A2  A1  A0

|   |   |   |

|   |   |   |

----------------

| Carry Look-  |

| Ahead Logic  |

----------------

|   |   |   |

C1  C2  C3  C4

|   |   |   |

-------------------------

| 4 Full Adders (FA0–FA3) |

-------------------------

|   |   |   |

S3  S2  S1  S0

The Carry Look-Ahead Logic computes all carry outputs (C₁, C₂, C₃, C₄) simultaneously, reducing propagation delay.

Components of a CLA

A 4-bit Carry Look-Ahead Adder consists of:

- Four Full Adders

- Carry Look-Ahead Logic Circuit

- Generate (G) Circuit

- Propagate (P) Circuit

Generate and Propagate Signals

For each bit position:

Generate Signal (G)

A carry is generated when both input bits are 1.

Gi=Ai×Bi

Propagate Signal (P)

A carry is propagated when exactly one input bit is 1.

Pi=AiBi

where:

- Ai= First input bit

- Bi= Second input bit

- = XOR operation

Carry Equations

The carry outputs are calculated as follows:

First Carry

C1=G0+P0C0

Second Carry

C2=G1+P1G0+P1P0C0

Third Carry

C3=G2+P2G1+P2P1G0+P2P1P0C0

Fourth Carry

C4=G3+P3G2+P3P2G1+P3P2P1G0+P3P2P1P0C0

These equations allow all carries to be calculated simultaneously without waiting for the previous carry.

Sum Equations

The sum outputs are calculated using:

Si=PiCi

where:

- Si= Sum bit

- Pi= Propagate bit

- Ci= Carry bit

Working of a 4-bit CLA

Example

Add the following binary numbers:

A = 1010

B = 0110

Input Carry (C₀) = 0

Step 1: Calculate Propagate (P)

A = 1010

B = 0110

-------------

P = 1100

Step 2: Calculate Generate (G)

A = 1010

B = 0110

-------------

G = 0010

Step 3: Calculate Carries

Using the carry equations:

C₁ = G₀ + P₀C₀

C₂ = G₁ + P₁G₀ + P₁P₀C₀

C₃ = G₂ + P₂G₁ + P₂P₁G₀ + P₂P₁P₀C₀

C₄ = G₃ + P₃G₂ + P₃P₂G₁ + P₃P₂P₁G₀ + P₃P₂P₁P₀C₀

All carry bits are obtained simultaneously.

Step 4: Calculate Sum

S = P XOR Carry

Final Result:

1010

0110

---------

10000

Therefore,

10 + 6 = 16

Working Steps of CLA

- Read the two binary numbers.

- Calculate Generate (G) signals.

- Calculate Propagate (P) signals.

- Compute all carry bits simultaneously.

- Calculate the sum bits.

- Display the final binary sum.

Advantages of Carry Look-Ahead Adder

- Very high-speed addition.

- Low propagation delay.

- Faster than Ripple Carry Adder.

- Improves processor performance.

- Suitable for large binary additions.

- Used in modern high-speed CPUs.

Disadvantages of Carry Look-Ahead Adder

- Complex circuit design.

- Requires more hardware.

- Higher implementation cost.

- Increased power consumption.

- Not economical for very small adders.

Applications

- Arithmetic Logic Unit (ALU)

- Intel and AMD processors

- Digital Signal Processing (DSP)

- High-speed microprocessors

- Scientific computing

- Embedded systems

- Supercomputers

Comparison Between CLA and Ripple Carry Adder

| Feature | Carry Look-Ahead Adder (CLA) | Ripple Carry Adder (RCA) |
| --- | --- | --- |
| Speed | Very High | Slow |
| Carry Calculation | Simultaneous | Sequential |
| Propagation Delay | Very Low | High |
| Hardware Requirement | More | Less |
| Circuit Complexity | High | Low |
| Cost | High | Low |
| Performance | Excellent | Moderate |
| Applications | Modern CPUs | Small digital circuits |

Difference Between CLA and RCA

Carry Look-Ahead Adder

- High-speed operation

- Carry calculated in parallel

- Suitable for high-performance processors

- More hardware and complex design

Ripple Carry Adder

- Simple circuit

- Carry propagates from one stage to another

- Suitable for small adders

- Slower due to carry delay

Conclusion

The Carry Look-Ahead Adder (CLA) is a high-speed binary adder that overcomes the delay problem of the Ripple Carry Adder by computing carry bits in advance using Generate (G) and Propagate (P) signals. This significantly improves the speed of arithmetic operations, making the CLA ideal for modern processors, Arithmetic Logic Units (ALUs), and high-performance digital systems. Although it requires more hardware and has a more complex design, its speed advantage makes it one of the most widely used adders in computer architecture.

Question 3

Using Booth's Algorithm, perform the multiplication of (-6) × (+5). Show all the steps involved in the multiplication process.

Answer

Introduction

Booth's Algorithm is a fast and efficient algorithm used to multiply signed binary numbers represented in 2's complement form. It was developed by Andrew Donald Booth to reduce the number of addition and subtraction operations during multiplication.

Unlike ordinary binary multiplication, Booth's Algorithm can handle both positive and negative numbers directly. It is widely used in modern processors, Arithmetic Logic Units (ALUs), and Digital Signal Processors (DSPs).

Objectives of Booth's Algorithm

- Multiply signed binary numbers.

- Reduce the number of arithmetic operations.

- Increase processor speed.

- Efficiently handle consecutive 1's in the multiplier.

- Simplify hardware implementation.

Given Data

Multiplicand (M) = −6

Multiplier (Q) = +5

Number of bits = 4

Step 1: Convert Decimal Numbers to Binary

Multiplicand = −6

First, convert +6 into binary.

+6 = 0110

Find the 2's complement to obtain −6.

1's Complement

0110

↓

1001

Add 1

1001

+0001

------

1010

Therefore,

M = -6 = 1010

Find −M

Since M = −6,

−M = +6 = 0110

Multiplier = +5

Q = 0101

Initial Register Values

| Register | Value |
| --- | --- |
| A (Accumulator) | 0000 |
| Q (Multiplier) | 0101 |
| M (Multiplicand) | 1010 |
| Q₋₁ | 0 |
| Number of Bits | 4 |

Booth's Decision Rules

| Q₀ Q₋₁ | Operation |
| --- | --- |
| 00 | No operation |
| 01 | A = A + M |
| 10 | A = A − M |
| 11 | No operation |

After every operation, perform an Arithmetic Right Shift (ARS).

Iteration 1

Current values:

A = 0000

Q = 0101

Q₋₁ = 0

Q₀Q₋₁ = 10

According to Booth's rule:

A = A − M

Since −M = 0110,

0000

+0110

------

0110

Now perform Arithmetic Right Shift.

Before Shift

A = 0110

Q = 0101

Q₋₁ = 0

After Shift

A = 0011

Q = 0010

Q₋₁ = 1

Iteration 2

Current values

A = 0011

Q = 0010

Q₋₁ = 1

Q₀Q₋₁ = 01

Operation

A = A + M

0011

1010

------

1101

Arithmetic Right Shift

Before Shift

A = 1101

Q = 0010

Q₋₁ = 1

After Shift

A = 1110

Q = 1001

Q₋₁ = 0

Iteration 3

Current values

A = 1110

Q = 1001

Q₋₁ = 0

Q₀Q₋₁ = 10

Operation

A = A − M

1110

+0110

------

0100

Arithmetic Right Shift

Before Shift

A = 0100

Q = 1001

Q₋₁ = 0

After Shift

A = 0010

Q = 0100

Q₋₁ = 1

Iteration 4

Current values

A = 0010

Q = 0100

Q₋₁ = 1

Q₀Q₋₁ = 01

Operation

A = A + M

0010

1010

------

1100

Arithmetic Right Shift

Before Shift

A = 1100

Q = 0100

Q₋₁ = 1

After Shift

A = 1110

Q = 0010

Q₋₁ = 0

Final Product

Combine A and Q:

A = 1110

Q = 0010

Final binary product:

11100010

Verification

Convert the result to decimal.

Take the 2's complement of 11100010:

- 1's complement:

00011101

- Add 1:

00011110

Decimal value:

30

Since the original binary number was negative,

11100010₂ = -30₁₀

Therefore,

(-6) × (+5) = -30

Hence, the multiplication is correct.

Flowchart of Booth's Algorithm (Text Representation)

Start

│

Initialize A, Q, M, Q₋₁

│

Check Q₀Q₋₁

│

┌───────────────┐

│00 or 11 → No Operation│

│01 → A = A + M         │

│10 → A = A − M         │

└───────────────┘

│

Arithmetic Right Shift

│

Decrease Count

│

Count = 0 ?

│

Yes → Product = AQ

│

Stop

Advantages of Booth's Algorithm

- Efficient for signed binary multiplication.

- Reduces the number of additions and subtractions.

- Faster than ordinary binary multiplication.

- Handles both positive and negative numbers.

- Suitable for high-speed processors.

- Reduces hardware complexity for large multipliers.

Disadvantages

- More complex than simple binary multiplication.

- Requires arithmetic right shifts.

- Needs additional control logic.

- Not efficient for very small multiplication problems.

Applications

- Arithmetic Logic Units (ALUs)

- Microprocessors

- Digital Signal Processors (DSP)

- Computer Architecture

- Embedded Systems

- Graphics Processing Units (GPUs)

- Scientific Computing

Conclusion

Booth's Algorithm is one of the most efficient methods for multiplying signed binary numbers using 2's complement representation. It reduces the number of arithmetic operations by examining pairs of bits in the multiplier and performing addition, subtraction, or no operation accordingly. In this example, multiplying −6 × +5 correctly produces the binary result 11100010₂, which is equal to −30₁₀. Due to its speed and efficiency, Booth's Algorithm is widely used in modern processors and digital systems.

Question 4

Explain the Restoring and Non-Restoring Division algorithms. Divide 13 by 3 using both methods and compare their performance.

Answer

Introduction

Division is one of the basic arithmetic operations performed in digital computers. Binary division is implemented using special algorithms in the Arithmetic Logic Unit (ALU) of a processor. The two most commonly used division algorithms are:

- Restoring Division Algorithm

- Non-Restoring Division Algorithm

Both algorithms repeatedly subtract the divisor from the dividend to obtain the quotient and remainder. The main difference is that the Restoring algorithm restores the accumulator whenever a subtraction results in a negative value, whereas the Non-Restoring algorithm avoids this restoration step, making it faster.

Restoring Division Algorithm

Definition

The Restoring Division Algorithm is a binary division method in which the previous value of the accumulator is restored whenever the subtraction result becomes negative.

Algorithm Steps

- Initialize:

  - Accumulator (A) = 0

  - Dividend (Q)

  - Divisor (M)

- Shift the combined register (A,Q) one bit to the left.

- Perform A = A − M.

- If A ≥ 0, set the least significant bit of Q (Q₀) to 1.

- If A < 0, restore A by adding M back and set Q₀ = 0.

- Repeat the process for all bits.

- After the final iteration:

  - Q contains the Quotient

  - A contains the Remainder

Given

Dividend = 13

Divisor = 3

Binary Representation

Dividend (Q) = 1101

Divisor (M) = 0011

Accumulator (A) = 0000

Restoring Division Working

Initial Values

| Register | Value |
| --- | --- |
| A | 0000 |
| Q | 1101 |
| M | 0011 |

Iteration 1

Shift Left (A,Q)

0000 1101

↓

0001 1010

Subtract Divisor

0001

-0011

------

1110

Result is negative.

Restore:

1110

+0011

------

0001

Set

Q₀ = 0

Iteration 2

Shift Left

0001 1010

↓

0011 0100

Subtract

0011

-0011

------

0000

Result is positive.

Therefore,

Q₀ = 1

Iteration 3

Shift Left

0000 0101

↓

0000 1010

Subtract

0000

-0011

------

1101

Negative result.

Restore

1101

+0011

------

0000

Set

Q₀ = 0

Iteration 4

Shift Left

0000 1010

↓

0001 0100

Subtract

0001

-0011

------

1110

Negative result.

Restore

1110

+0011

------

0001

Set

Q₀ = 0

Final Result (Restoring Division)

| Output | Value |
| --- | --- |
| Quotient | 0100 = 4 |
| Remainder | 0001 = 1 |

Therefore,

13 ÷ 3 = 4, Remainder = 1

Non-Restoring Division Algorithm

Definition

The Non-Restoring Division Algorithm is an improved version of restoring division. Instead of restoring the accumulator after a negative subtraction, it performs an addition in the next step if the accumulator is negative.

This eliminates unnecessary restoration operations and increases the speed of execution.

Algorithm Steps

- Initialize A, Q, and M.

- Shift (A,Q) left by one bit.

- If A is positive, perform A = A − M.

- If A is negative, perform A = A + M.

- Set the quotient bit according to the sign of A.

- Repeat until all bits are processed.

- If the final A is negative, restore it once by adding M.

Working

Initial Values

| Register | Value |
| --- | --- |
| A | 0000 |
| Q | 1101 |
| M | 0011 |

Iteration 1

Shift Left

0000 1101

↓

0001 1010

Subtract Divisor

0001

-0011

------

1110

Negative

Q₀ = 0

Iteration 2

Since A is negative,

Shift Left

Perform Addition

1110

+0011

------

0001

Positive

Q₀ = 1

Iteration 3

Shift Left

Subtract Divisor

0010

-0011

------

1111

Negative

Q₀ = 0

Iteration 4

Shift Left

Since accumulator is negative,

Add Divisor

1111

+0011

------

0010

Positive

Q₀ = 0

Final Result (Non-Restoring Division)

| Output | Value |
| --- | --- |
| Quotient | 0100 = 4 |
| Remainder | 0001 = 1 |

Therefore,

13 ÷ 3 = 4, Remainder = 1

Comparison Between Restoring and Non-Restoring Division

| Feature | Restoring Division | Non-Restoring Division |
| --- | --- | --- |
| Restoration Step | Required | Not Required |
| Speed | Slower | Faster |
| Arithmetic Operations | More | Fewer |
| Hardware Complexity | Simple | Slightly Complex |
| Performance | Lower | Higher |
| Processor Usage | Older Systems | Modern Systems |

Advantages of Restoring Division

- Easy to understand and implement.

- Simple hardware design.

- Produces accurate results.

- Suitable for learning binary division.

Advantages of Non-Restoring Division

- Faster than restoring division.

- Reduces the number of arithmetic operations.

- Better processor performance.

- Efficient for high-speed processors.

- Widely used in modern computer systems.

Applications

- Arithmetic Logic Unit (ALU)

- Microprocessors

- Embedded Systems

- Digital Signal Processing (DSP)

- Computer Architecture

- Scientific Computing

- Digital Computers

Conclusion

The Restoring and Non-Restoring Division algorithms are important methods used to perform binary division in digital computers. The Restoring Division Algorithm restores the accumulator whenever the subtraction result becomes negative, making it simpler but slower. The Non-Restoring Division Algorithm avoids repeated restoration, resulting in fewer arithmetic operations and better execution speed.

For the given example:

- Dividend = 13

- Divisor = 3

Both algorithms correctly produce:

- Quotient = 4

- Remainder = 1

Because of its improved efficiency and reduced execution time, the Non-Restoring Division Algorithm is preferred in modern processors and high-performance computing systems.

This answer is suitable for approximately 2–3 handwritten pages in a 30-mark university examination.

Question 5

Represent the decimal number (−13.25) in IEEE 754 floating-point format. Explain the IEEE 754 standard and perform floating-point addition with a suitable example.

Answer

Introduction

The IEEE 754 Standard is an internationally accepted standard for representing floating-point (real) numbers in computers. It was developed by the Institute of Electrical and Electronics Engineers (IEEE) to ensure a common format for storing and performing arithmetic operations on decimal numbers.

Unlike integers, floating-point numbers can represent very large, very small, and fractional values. The IEEE 754 standard is widely used in processors, calculators, graphics systems, scientific computing, artificial intelligence, and engineering applications.

IEEE 754 Floating-Point Representation

A floating-point number is divided into three fields:

- Sign Bit (S)

- Exponent (E)

- Mantissa (Fraction/Significand)

Single Precision (32-bit)

| Field | Number of Bits |
| --- | --- |
| Sign | 1 |
| Exponent | 8 |
| Mantissa | 23 |

--------------------------------------------------------

| Sign | Exponent (8 bits) | Mantissa (23 bits) |

--------------------------------------------------------

Double Precision (64-bit)

| Field | Number of Bits |
| --- | --- |
| Sign | 1 |
| Exponent | 11 |
| Mantissa | 52 |

--------------------------------------------------------------------

| Sign | Exponent (11 bits) | Mantissa (52 bits) |

--------------------------------------------------------------------

Advantages of IEEE 754

- Represents positive and negative real numbers.

- High precision.

- Common worldwide standard.

- Supports very large and very small values.

- Provides special values like Infinity and NaN (Not a Number).

- Used by almost all modern processors.

Representation of −13.25

Step 1: Convert the Integer Part to Binary

Decimal:

13

Binary:

13 = 1101₂

Step 2: Convert the Fractional Part

Fraction:

0.25

Multiply repeatedly by 2.

| Multiplication | Integer Part |
| --- | --- |
| 0.25 × 2 = 0.50 | 0 |
| 0.50 × 2 = 1.00 | 1 |

Therefore,

0.25 = 0.01₂

Hence,

13.25 = 1101.01₂

Step 3: Normalize the Binary Number

Move the binary point after the first digit.

1101.01₂

=

1.10101 × 2³

Exponent = 3

Step 4: Determine the Sign Bit

Since the number is negative,

Sign = 1

Step 5: Calculate the Exponent

IEEE 754 Single Precision uses a bias of 127.

Actual exponent

3

Biased exponent

127 + 3 = 130

Convert 130 to binary.

130 = 10000010₂

Step 6: Find the Mantissa

Normalized binary number

1.10101 × 2³

The leading 1 is omitted.

Mantissa

10101000000000000000000

(23 bits)

Final IEEE 754 Single Precision Representation

| Field | Binary Value |
| --- | --- |
| Sign | 1 |
| Exponent | 10000010 |
| Mantissa | 10101000000000000000000 |

Final 32-bit representation:

1 10000010 10101000000000000000000

IEEE 754 Double Precision Representation

Bias = 1023

Exponent

1023 + 3 = 1026

Binary

1026 = 10000000010₂

Mantissa

1010100000000000000000000000000000000000000000000000

Final 64-bit representation:

1

10000000010

1010100000000000000000000000000000000000000000000000

Floating-Point Addition

Example

Add

5.5 + 2.25

Step 1: Convert to Binary

5.5 = 101.1₂

2.25 = 10.01₂

Step 2: Normalize

101.1₂

=

1.011 × 2²

10.01₂

=

1.001 × 2¹

Step 3: Equalize Exponents

Shift the second number.

1.001 × 2¹

↓

0.1001 × 2²

Now both numbers have exponent 2.

Step 4: Add Mantissas

1.0110

0.1001

--------

1.1111

Step 5: Normalize the Result

1.1111 × 2²

No further normalization is required.

Step 6: Convert Back to Decimal

111.11₂

=

7.75₁₀

Therefore,

5.5 + 2.25 = 7.75

Applications of IEEE 754

- Scientific Computing

- Engineering Applications

- Artificial Intelligence

- Machine Learning

- Computer Graphics

- Financial Calculations

- Weather Forecasting

- Digital Signal Processing (DSP)

- Computer Architecture

Advantages

- Standardized representation worldwide.

- High accuracy for real numbers.

- Supports very small and very large values.

- Handles positive and negative numbers.

- Supports special values such as Infinity and NaN.

- Widely supported by modern processors.

Disadvantages

- Rounding errors may occur.

- Complex hardware implementation.

- Uses more memory than integer representation.

- Some decimal numbers cannot be represented exactly.

Comparison Between Single and Double Precision

| Feature | Single Precision | Double Precision |
| --- | --- | --- |
| Total Bits | 32 | 64 |
| Sign Bits | 1 | 1 |
| Exponent Bits | 8 | 11 |
| Mantissa Bits | 23 | 52 |
| Precision | Moderate | High |
| Memory Required | Less | More |
| Accuracy | Lower | Higher |

Conclusion

The IEEE 754 Floating-Point Standard provides a universal method for representing real numbers in binary using Sign, Exponent, and Mantissa fields. It ensures consistency, accuracy, and portability across different computer systems. In this problem, the decimal number −13.25 was successfully converted into both 32-bit (Single Precision) and 64-bit (Double Precision) IEEE 754 formats. A floating-point addition example (5.5 + 2.25 = 7.75) demonstrated the steps of exponent alignment, mantissa addition, normalization, and final conversion. Because of its precision and reliability, IEEE 754 is widely used in modern processors, scientific computing, artificial intelligence, engineering, and graphics applications.