SIMATS ENGINEERING

Department of Computer Science & Engineering

ASSESSMENT TOOL 2-DEBUGGING & OPTIMISATION TASK

| Course Code: | CSA12 | Course Title: | Computer Architecture |
| --- | --- | --- | --- |
| CO Assessed: | CO2 | Assessment: | ASSESSMENT TOOL3- DEBUGGING & OPTIMISATION TASK |
| Weightage: | 20% | Total Marks: | 25 |
| CO2 Statement: | Apply integer and floating-point arithmetic algorithms including Booth's multiplication, restoring/non-restoring division, and IEEE 754 standard operations to solve fixed-point and floating-point computational problems. (BL3) |  |  |
| Student Name: |  | Reg.No.: |  |
| Department: |  | Date: |  |

ANNEXURE A

1. A program performing signed integer addition using 2’s complement produces  incorrect results when adding large values (e.g., +120 and +50 in 8-bit representation).  Debug the issue by analyzing the binary computation and identifying whether overflow  has occurred. Propose a solution to correctly detect and handle overflow in such  cases.

2. A processor using a ripple carry adder shows significant delay in executing arithmetic  operations in a real-time system. Debug the performance bottleneck by examining how  carry propagates through each bit. Suggest an optimized solution using a carry look ahead adder, and explain how it reduces delay.

3. A multiplication unit implementing Booth’s algorithm gives incorrect results for certain  negative operands. Debug the step-by-step execution of the algorithm, focusing on bit pair checking, arithmetic shifts, and sign extension. Identify the possible source of  error and suggest corrections to ensure accurate signed multiplication.

4. An embedded system using the restoring division algorithm experiences inefficiency  due to repeated restoration steps, leading to increased execution time. Debug the  algorithm’s workflow and identify where unnecessary operations occur. Propose an  optimized approach using the non-restoring division method, explaining how it  improves performance.

5. A scientific application using IEEE 754 single precision floating-point representation  produces inaccurate results when adding very small and very large numbers. Debug  the issue by analyzing exponent alignment, normalization, and rounding errors.  Suggest optimization techniques, such as using double precision or algorithmic  adjustments, to improve accuracy.

Code of Conduct:

I DEEPTHI KORRAPATI (192572012) (Name / Reg No)  certify that this submission is my original work and that I have adhered to the  guidelines specified.

1. Signed Integer Addition using 2's Complement Overflow Detection

Introduction

Signed integers in computers are represented using the 2's complement method because it simplifies arithmetic operations. However, when two large signed numbers are added, the result may exceed the representable range, causing overflow.

Given Example

+120 + +50

Step 1: Convert into 8-bit Binary

+120 = 01111000

+50  = 00110010

Step 2: Binary Addition

01111000

+ 00110010

------------

10101010

Result:

10101010

This binary represents

-86

instead of

170

Representable Range

For 8-bit signed integers

-128 to +127

Actual answer

170

cannot be represented.

Hence,

Overflow has occurred.

Overflow Detection

Overflow occurs when

Two positive numbers produce a negative result.

Two negative numbers produce a positive result.

Overflow Rule

Carry into MSB ≠ Carry out of MSB

OR

Same sign operands

↓

Different sign result

↓

Overflow

Truth Table

| Operand A | Operand B | Result | Overflow |
| --- | --- | --- | --- |
| + | + | - | Yes |
| - | - | + | Yes |
| + | - | Correct | No |
| - | + | Correct | No |

Binary Analysis

Operand1 MSB = 0

Operand2 MSB = 0

Result MSB = 1

Positive + Positive = Negative

Overflow detected.

Possible Causes

Fixed 8-bit register size

Arithmetic exceeds +127

No overflow flag checking

Solution

Method 1

Use Overflow Flag (V flag)

Processor checks

Carry into MSB

Carry out of MSB

If different

V = 1

Method 2

Increase register size

Example

16-bit

32-bit

64-bit

Method 3

Exception Handling

If overflow occurs

Display Error

OR

Saturate output

(+127 or -128)

Advantages

Prevents incorrect calculations

Reliable arithmetic

Used in processors

Improves program correctness

Conclusion

Overflow occurs because the sum exceeds the representable range of an 8-bit signed number. Detecting overflow using the overflow flag or sign-bit comparison ensures correct arithmetic and prevents incorrect program execution.

2. Ripple Carry Adder Delay and Carry Look Ahead Adder

Introduction

Ripple Carry Adder (RCA) adds binary numbers one bit at a time. The carry generated in one stage must propagate to the next stage, making it slow for high-speed applications.

Working of Ripple Carry Adder

Example

1011

+

0110

Each full adder waits for the previous carry.

FA0 → FA1 → FA2 → FA3

Carry ripples through every stage.

Performance Bottleneck

Suppose each Full Adder delay

2 ns

For 8 bits

Delay

=

8 × 2

=

16 ns

Large delay reduces processor speed.

Problem

Carry propagation

Bit0

↓

Bit1

↓

Bit2

↓

Bit3

↓

...

Every stage waits.

Carry Look Ahead Adder (CLA)

Instead of waiting,

CLA predicts carry.

Uses

Generate

Gi = AiBi

Propagate

Pi = Ai XOR Bi

Carry equations

C1 = G0 + P0C0

C2 = G1 + P1G0 + P1P0C0

C3 = G2 + P2G1 + P2P1G0 + P2P1P0C0

Carries are calculated simultaneously.

Comparison

| Ripple Carry | Carry Look Ahead |
| --- | --- |
| Sequential carry | Parallel carry |
| Slow | Fast |
| Delay increases with bits | Low delay |
| Simple | Slightly complex |

Advantages

Faster arithmetic

High-speed processors

Reduced propagation delay

Better real-time performance

Applications

CPUs

ALUs

DSP Processors

High-speed computing

Conclusion

Carry Look Ahead Adders eliminate the ripple effect by generating carries in parallel, significantly reducing delay and improving processor performance.

3. Debugging Booth's Multiplication Algorithm

Introduction

Booth's Algorithm efficiently performs signed binary multiplication using 2's complement representation. It reduces the number of addition and subtraction operations.

Registers Used

A

Accumulator

Q

Multiplier

M

Multiplicand

Q-1

Extra Bit

Booth Decision Table

| Q0 | Q-1 | Operation |
| --- | --- | --- |
| 00 | No operation |  |
| 01 | A=A+M |  |
| 10 | A=A−M |  |
| 11 | No operation |  |

Algorithm Steps

Initialize registers

Check (Q0,Q−1)

Perform operation

Arithmetic right shift

Repeat n times

Common Errors

Error 1

Incorrect bit pair checking

Wrong

10 treated as 01

Wrong multiplication.

Error 2

Logical shift instead of Arithmetic shift

Wrong

0 inserted

Correct

Sign bit copied

Error 3

Improper sign extension

Negative numbers become positive.

Error 4

Wrong subtraction

Need

2's complement

of multiplicand

Debugging Steps

Verify

✔ Bit pair

✔ Arithmetic shift

✔ Sign extension

✔ Register update

✔ Loop count

Corrections

Use arithmetic right shift

Preserve sign bit

Check Q0 and Q−1 correctly

Correct 2's complement subtraction

Perform exactly n iterations

Advantages

Efficient signed multiplication

Fewer additions

Handles negative numbers

Faster than repeated addition

Conclusion

Correct bit-pair checking, arithmetic shifts, and sign extension are essential for accurate signed multiplication using Booth's Algorithm.

4. Restoring Division vs Non-Restoring Division

Introduction

Restoring Division performs binary division by repeatedly subtracting the divisor. If subtraction produces a negative remainder, the algorithm restores the previous value, increasing execution time.

Restoring Division Steps

Shift

Subtract divisor

Check sign

If negative

Restore remainder

Set quotient bit

Repeat

Problem

Every unsuccessful subtraction requires

Restoration

↓

Extra addition

↓

Extra clock cycles

Hence slower.

Unnecessary Operations

Repeated

Subtract

↓

Restore

↓

Subtract

↓

Restore

Large execution time.

Non-Restoring Division

Instead of restoring,

Next operation depends on remainder sign.

Positive

Subtract

Negative

Add

No restoration required.

Algorithm

If

R ≥ 0

Subtract divisor

Else

Add divisor

Repeat until completion.

Comparison

| Restoring | Non-Restoring |
| --- | --- |
| Restore every negative remainder | No restoration |
| More clock cycles | Fewer cycles |
| Slower | Faster |
| Simple | Efficient |

Advantages

Faster division

Fewer arithmetic operations

Better processor utilization

Reduced execution time

Applications

Microprocessors

Embedded systems

DSP

Digital arithmetic units

Conclusion

Non-restoring division removes unnecessary restoration operations, reducing execution time and improving arithmetic performance in embedded systems.

5. IEEE 754 Floating Point Addition Errors and Optimization

Introduction

IEEE 754 floating-point format stores real numbers using

Sign bit

Exponent

Mantissa

It provides a wide range of values but introduces rounding and precision errors when adding numbers with very different magnitudes.

IEEE 754 Single Precision Format

| Field | Bits |
| --- | --- |
| Sign | 1 |
| Exponent | 8 |
| Mantissa | 23 |

Total

32 bits

Problem Example

100000000

+

0.000001

Expected

100000000.000001

Actual

100000000

Small value disappears.

Reason

Exponent Alignment

Smaller number's mantissa is shifted right many positions to match the larger exponent.

Example

Large exponent

↓

Small mantissa shifted

↓

Lost precision

Normalization

After addition

Binary point adjusted

Mantissa normalized.

This may remove additional least significant bits.

Rounding Errors

IEEE uses

Round to nearest

Small bits may be discarded.

Possible Errors

Precision loss

Cancellation error

Rounding error

Underflow

Overflow

Optimization Techniques

1. Double Precision

Use

64-bit

instead of

32-bit

Higher accuracy.

2. Kahan Summation Algorithm

Reduces accumulated rounding errors during repeated additions.

3. Add Small Numbers First

Sorting operands by magnitude minimizes loss of precision.

4. Higher Precision Libraries

Use extended or arbitrary-precision arithmetic for scientific applications.

5. Error Checking

Detect

Overflow

Underflow

NaN

Infinity

before computation.

Advantages of Optimization

Improved numerical accuracy

Reduced rounding errors

Better scientific computations

Reliable engineering calculations

Applications

Scientific computing

Artificial Intelligence

Graphics processing

Weather forecasting

Financial analysis

Conclusion

IEEE 754 floating-point arithmetic is efficient but susceptible to precision loss when adding numbers with widely different magnitudes. Using double precision, improved summation algorithms, and proper error handling significantly enhances accuracy and reliability in scientific applications.

for this assessment.I understand that any violation of academic  integrity rules will result in disciplinary action.

Signature of the student:

MARKS ALLOCATION

|  | Identification of Errors / Bugs(20%) | Debugging Approach & Analysis(20%) | Correctnes s of Debugged Solution(2 5%) | Optimization Techniques Applied(20%) | Testing and Documentation(15 %) |
| --- | --- | --- | --- | --- | --- |
| Q1 |  |  |  |  |  |
| Q2 |  |  |  |  |  |
| Q3 |  |  |  |  |  |
| Q4 |  |  |  |  |  |
| Q5 |  |  |  |  |  |

RUBRICS FOR CALCULATION:

| Criteria |  | Weightage Level 4 – Exemplary | Level 3 – Proficient | Level 2 – Developing | Level 1 – Beginning |
| --- | --- | --- | --- | --- | --- |
| Identification of Errors / Bugs | 20% | Accurately identifies all errors and issues with complete understanding of the problem | Identifies most errors with minor omissions | Identifies limited errors; misses key issues | Unable to identify errors or misunderstands the problem |
| Debugging Approach & Analysis | 20% | Uses effective debugging techniques with clear analysis and root cause identification | Applies suitable debugging methods with minor gaps | Uses basic debugging methods with limited analysis | No systematic debugging approach followed |
| Correctness of Debugged Solution | 25% | Provides completely corrected solution with accurate functionality and expected output | Provides working solution with minor errors | Partially correct solution with limited functionality | Incorrect solution or fails to resolve errors |
| Optimization Techniques Applied | 20% | Applies efficient optimization methods to improve performance, memory usage and quality | Performs optimization with noticeable improvements | Limited optimization with minimal improvements | No optimization applied or inefficient implementation |
| Testing and Documentation | 15% | Performs complete testing with proper validation and clear documentation | Provides adequate testing and documentation with minor issues | Limited testing and incomplete documentation | No proper testing or documentation provided |