SIMATS ENGINEERING

Department of Computer Science & Engineering

ASSESSMENT TOOL 2-PSEUDOCODE WRITING TASK (ALGORITHM DESIGN)

| Course Code: | CSA12 | Course Title: | Computer Architecture |
| --- | --- | --- | --- |
| CO Assessed: | CO5 | Assessment: | ASSESSMENT TOOL2-PSEUDOCODE WRITING TASK (ALGORITHM DESIGN) |
| Weightage: | 40% | Total Marks: | 25 |
| CO5 Statement: | Construct I/O communication mechanisms by comparing memory-mapped and I/O-mapped techniques, interrupt handling (vectored, hardware, software), DMA controllers, and advanced bus interfaces including PCIe, USB, NVMe, and Thunderbolt. |  |  |
| Student Name: |  | Reg.No.: |  |
| Department: |  | Date: |  |

ANNEXURE A

Pseudocode Writing Task (Algorithm Design)

- Write pseudocode to design a DMA-based data transfer algorithm that moves data from an I/O device to memory while handling interrupts and minimizing CPU involvement.

- Develop pseudocode for an interrupt handling system that prioritizes vectored interrupts from multiple devices and dispatches appropriate service routines.

- Write pseudocode to select between memory-mapped I/O and I/O-mapped I/O dynamically based on latency and bandwidth requirements.

- Design pseudocode for a bus arbitration algorithm that manages multiple devices communicating over PCIe, USB, and Thunderbolt interfaces.

- Write pseudocode to implement a hybrid I/O communication mechanism that uses polling for low-speed devices and DMA with interrupts for high-speed devices.

Code of Conduct:

I DEEPTHI KORRAPATI(192572012) (Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student:

MARKS ALLOCATION

|  | Problem Understanding(20%) | Algorithm Design(30%) | Use of Control Structures(20%) | Pseudocode Structure(10%) | Completeness(20%) |
| --- | --- | --- | --- | --- | --- |
| Q1 |  |  |  |  |  |
| Q2 |  |  |  |  |  |
| Q3 |  |  |  |  |  |
| Q4 |  |  |  |  |  |
| Q5 |  |  |  |  |  |

RUBRICS FOR CALCULATION:

| Criteria | Weightage | Level 4 – Exemplary | Level 3 – Proficient | Level 2 – Developing | Level 1 – Beginning |
| --- | --- | --- | --- | --- | --- |
| Problem Understanding | 20% | Identifies all inputs, outputs, constraints accurately | Minor missing elements | Partial understanding | Incorrect interpretation |
| Algorithm Design | 30% | Complete, correct, and logically sound solution | Minor logical gaps | Partially correct logic | Incorrect approach |
| Use of Control Structures | 20% | Efficient and appropriate use of loops, conditions, functions/modules | Minor inefficiencies | Limited or basic usage | Incorrect or missing constructs |
| Pseudocode Structure | 10% | Well-structured, clear, consistent format, easy to follow | Mostly clear with minor issues | Difficult to follow in parts | Poorly structured and unclear |
| Completeness | 20% | Handles all cases; optimized approach | Handles most cases; reasonable efficiency | Handles basic cases only | Incomplete or inefficient |

Q1. Write pseudocode to design a DMA-based data transfer algorithm that moves data from an I/O device to memory while handling interrupts and minimizing CPU involvement.

1. Introduction

Direct Memory Access (DMA) is an I/O communication technique in which a DMA controller transfers data directly between an I/O device and main memory without requiring the CPU to move every individual byte or word. The CPU is involved mainly in setting up the transfer, while the DMA controller performs the actual data movement. After the transfer is completed, the DMA controller generates an interrupt to inform the CPU.

The main objective of the algorithm is to minimize CPU involvement while ensuring that the data is transferred correctly, the device is serviced, and the CPU receives a notification after completion. The algorithm therefore uses initialization, DMA transfer, status checking, interrupt generation, and interrupt service handling.

2. Inputs

• I/O device identifier
• Starting memory address
• Number of bytes/words to transfer
• DMA controller
• Transfer direction
• Device status
• DMA status and interrupt status

3. Outputs

• Data transferred from the I/O device to memory
• DMA completion status
• Interrupt notification to the CPU
• Error status if the transfer fails

4. Pseudocode

BEGIN

INITIALIZE DMA_CONTROLLER
    INITIALIZE INTERRUPT_SYSTEM

INPUT DEVICE_ID
    INPUT MEMORY_START_ADDRESS
    INPUT TRANSFER_SIZE

CHECK device status

IF device is NOT READY THEN
        WAIT until device becomes READY
    END IF

DMA_CONTROLLER.SOURCE ← DEVICE_ID
    DMA_CONTROLLER.DESTINATION ← MEMORY_START_ADDRESS
    DMA_CONTROLLER.COUNT ← TRANSFER_SIZE
    DMA_CONTROLLER.DIRECTION ← DEVICE_TO_MEMORY
    DMA_CONTROLLER.STATUS ← READY
    DMA_CONTROLLER.INTERRUPT_ENABLE ← TRUE

CPU sends START command to DMA_CONTROLLER

WHILE DMA_CONTROLLER.TRANSFER_COMPLETE = FALSE
        DMA_CONTROLLER.REQUEST_BUS()

IF bus is granted THEN

READ data from I/O device

WRITE data to memory

UPDATE MEMORY_ADDRESS
            DECREMENT TRANSFER_COUNT

IF TRANSFER_COUNT = 0 THEN
                DMA_CONTROLLER.TRANSFER_COMPLETE ← TRUE
            END IF

RELEASE_BUS()

ELSE
            WAIT for bus grant
        END IF

IF DMA_CONTROLLER.ERROR = TRUE THEN
            GENERATE DMA_ERROR_INTERRUPT
            EXIT LOOP
        END IF
    END WHILE

IF DMA_CONTROLLER.TRANSFER_COMPLETE = TRUE THEN
        GENERATE DMA_COMPLETE_INTERRUPT
    END IF

CPU continues other processing while DMA transfer is active

WHEN INTERRUPT_RECEIVED DO
        SAVE CPU CONTEXT
        READ DMA_STATUS

IF DMA_STATUS = COMPLETE THEN
            CLEAR DMA_INTERRUPT
            RECORD successful transfer
        ELSE IF DMA_STATUS = ERROR THEN
            RECORD transfer error
            HANDLE ERROR
        END IF

CLEAR INTERRUPT_FLAG
        RESTORE CPU CONTEXT
        RETURN FROM INTERRUPT
    END WHEN

END

DMA-Based Data Transfer

START
  ↓
Initialize DMA + Interrupt System
  ↓
Enter Device, Memory Address, Transfer Size
  ↓
Is Device Ready?
 ┌───────────────┐
NO ↓             YES ↓
Wait          Configure DMA
                  ↓
             Start DMA Transfer
                  ↓
             Request Bus
                  ↓
             Bus Granted?
          ┌───────┴───────┐
         NO              YES
         ↓                 ↓
       Wait        Read I/O Device
                         ↓
                    Write to Memory
                         ↓
                    Update Address
                         ↓
                  Decrease Count
                         ↓
                    Count = 0?
                  ┌──────┴──────┐
                 NO             YES
                 ↓               ↓
            Continue        Generate Interrupt
                                 ↓
                         CPU Handles ISR
                                 ↓
                           Check Status
                                 ↓
                              END

5. Explanation of the Algorithm

First, the DMA controller and interrupt system are initialized. The CPU provides the DMA controller with the required information, including the source device, destination memory address, transfer size, and transfer direction. Once these parameters are configured, the CPU starts the DMA operation.

After initialization, the DMA controller requests access to the system bus. When access is granted, the controller reads data from the I/O device and writes it directly into memory. The memory address is updated after every transfer and the remaining transfer count is decreased.

The important feature of this algorithm is that the CPU does not have to execute a separate instruction for every data item. Therefore, CPU overhead is significantly reduced. The CPU can perform other useful computations while the DMA controller is transferring data.

When the transfer count reaches zero, the DMA controller marks the operation as complete and generates an interrupt. The CPU then temporarily stops its current work, saves its context, and executes the DMA interrupt service routine. The service routine checks the DMA status, clears the interrupt flag, records the result, and returns control to the interrupted program.

6. Interrupt Handling

Interrupts are important because they eliminate the need for the CPU to continuously poll the DMA controller. Instead of repeatedly asking whether the transfer is complete, the CPU receives an interrupt only when attention is required. This improves processor efficiency.

If an error occurs, the DMA controller generates an error interrupt. The CPU then executes an error-handling routine. Possible actions include recording the error, resetting the DMA controller, retrying the operation, or notifying the higher-level software.

7. Advantages

• Minimizes CPU involvement.
• Provides efficient high-speed data transfer.
• Allows CPU and I/O operations to overlap.
• Uses interrupts instead of continuous polling.
• Suitable for large blocks of data.
• Reduces CPU instruction overhead.
• Improves overall system throughput.

8. Conclusion

The proposed DMA-based pseudocode transfers data directly from an I/O device to memory and uses an interrupt to notify the CPU after completion. The CPU only performs initialization and interrupt handling instead of controlling every individual transfer. Hence, the algorithm satisfies the requirement of minimizing CPU involvement while maintaining reliable and efficient I/O communication.

Q2. Develop pseudocode for an interrupt handling system that prioritizes vectored interrupts from multiple devices and dispatches appropriate service routines.

1. Introduction

An interrupt is a signal that requests the CPU to temporarily stop its current execution and perform a specific service operation. In a computer system, several devices may generate interrupts at the same time. Therefore, an interrupt handling system must identify the source of the interrupt, determine its priority, and execute the correct Interrupt Service Routine (ISR).

A vectored interrupt system is useful because each interrupt source is associated with a vector or address that identifies its service routine. Instead of searching through all devices to determine the source, the CPU can use the interrupt vector to directly dispatch the appropriate ISR.

2. Inputs

• Interrupt requests from multiple devices
• Interrupt vector table
• Device priority values
• Device status registers
• CPU interrupt status

3. Outputs

• Correct ISR execution
• Serviced interrupt
• Updated device status
• Cleared interrupt request

4. Pseudocode

BEGIN

INITIALIZE INTERRUPT_CONTROLLER
    INITIALIZE INTERRUPT_VECTOR_TABLE

SET priority for each device
    ENABLE required interrupts

WHILE SYSTEM_RUNNING = TRUE

IF INTERRUPT_REQUEST_EXISTS = TRUE THEN

SAVE CPU_CONTEXT

DISABLE LOWER_PRIORITY_INTERRUPTS

COLLECT all pending interrupt requests

SORT pending requests by PRIORITY

SELECT highest priority interrupt

INTERRUPT_VECTOR ← GET_VECTOR(highest priority device)

ISR_ADDRESS ← VECTOR_TABLE[INTERRUPT_VECTOR]

ACKNOWLEDGE interrupt to controller

CALL ISR_ADDRESS

CLEAR interrupt request

RESTORE CPU_CONTEXT

ENABLE interrupts

ELSE
            CONTINUE current CPU operation
        END IF

END WHILE

END

Q2 – Vectored Interrupt Handling

START
  ↓
Initialize Interrupt Controller
  ↓
Enable Required Interrupts
  ↓
Interrupt Request?
 ┌──────────────┐
NO ↓            YES ↓
Continue      Save CPU Context
                  ↓
             Collect Pending Requests
                  ↓
             Determine Priority
                  ↓
             Select Highest Priority
                  ↓
             Obtain Interrupt Vector
                  ↓
             Locate ISR Address
                  ↓
             Execute ISR
                  ↓
             Clear Interrupt
                  ↓
             Restore CPU Context
                  ↓
                 END

5. Interrupt Service Routine Logic

WHEN ISR(device) IS CALLED DO

READ device status

IF device status indicates valid request THEN
        PERFORM required service operation
    END IF

CLEAR device interrupt flag
    SEND END_OF_INTERRUPT signal

RETURN

END ISR

6. Priority Handling

Suppose three devices generate interrupts simultaneously:

• Device A – High priority
• Device B – Medium priority
• Device C – Low priority

The interrupt controller first collects the pending requests. It compares their priority values and selects Device A. The CPU obtains the interrupt vector corresponding to Device A and jumps directly to its ISR.

After Device A is serviced, the controller checks whether other interrupt requests are still pending. Device B is then serviced, followed by Device C.

This priority mechanism ensures that important devices receive service before less critical devices.

7. Nested Interrupts

The system can also support nested interrupts. While a low-priority ISR is executing, a higher-priority interrupt may arrive. In that situation, the CPU can temporarily suspend the lower-priority ISR, save its context, and execute the higher-priority ISR.

Pseudocode:

IF CURRENT_ISR_PRIORITY < NEW_INTERRUPT_PRIORITY THEN

SAVE CURRENT_ISR_CONTEXT
    CALL HIGHER_PRIORITY_ISR
    RESTORE CURRENT_ISR_CONTEXT
    CONTINUE CURRENT_ISR

ELSE

STORE interrupt as PENDING

END IF

8. Vectored Interrupt Table

The vector table provides a direct association between interrupt sources and their service routines.

Example:

Device 1 → Vector 10 → ISR_Device1
Device 2 → Vector 20 → ISR_Device2
Device 3 → Vector 30 → ISR_Device3

When Device 2 generates an interrupt, the controller supplies Vector 20. The CPU uses this vector to locate ISR_Device2 without searching through all devices.

9. Advantages

• Fast identification of interrupt source.
• Supports multiple devices.
• Provides priority-based servicing.
• Reduces interrupt response time.
• Allows nested interrupt handling.
• Makes the system modular.
• Avoids unnecessary polling.

10. Conclusion

The proposed interrupt handling system collects pending interrupts, determines their priorities, obtains the corresponding interrupt vector, and dispatches the appropriate service routine. Vectored interrupts make the process faster because the CPU can directly locate the required ISR. Priority handling ensures that important devices are serviced first, improving the responsiveness and reliability of the computer system.

Q3. Write pseudocode to select between memory-mapped I/O and I/O-mapped I/O dynamically based on latency and bandwidth requirements.

1. Introduction

Memory-mapped I/O and I/O-mapped I/O are two techniques used by a processor to communicate with I/O devices. In memory-mapped I/O, I/O devices are assigned addresses within the normal memory address space. The processor can use normal memory instructions to access the devices. In I/O-mapped I/O, a separate I/O address space is used for device communication, generally with dedicated I/O instructions.

The choice between the two techniques can be made according to application requirements such as latency, bandwidth, address-space availability, transfer size, and device characteristics.

2. Inputs

• Required latency
• Required bandwidth
• Available memory address space
• Available I/O address space
• Device type
• Transfer size
• Processor capabilities

3. Outputs

• Selected I/O communication technique
• Configuration of the selected technique
• Communication mode used for the device

4. Pseudocode

BEGIN

INPUT REQUIRED_LATENCY
    INPUT REQUIRED_BANDWIDTH
    INPUT AVAILABLE_MEMORY_SPACE
    INPUT AVAILABLE_IO_SPACE
    INPUT TRANSFER_SIZE
    INPUT DEVICE_TYPE

IF REQUIRED_LATENCY <= LOW_LATENCY_LIMIT THEN

IF AVAILABLE_MEMORY_SPACE >= REQUIRED_ADDRESS_SPACE THEN
            SELECT MEMORY_MAPPED_IO
        ELSE
            SELECT IO_MAPPED_IO
        END IF

ELSE IF REQUIRED_BANDWIDTH >= HIGH_BANDWIDTH_LIMIT THEN

IF DEVICE_SUPPORTS_FAST_MEMORY_ACCESS = TRUE THEN
            SELECT MEMORY_MAPPED_IO
        ELSE
            SELECT IO_MAPPED_IO
        END IF

ELSE

IF TRANSFER_SIZE <= SMALL_TRANSFER_LIMIT THEN

IF AVAILABLE_IO_SPACE > 0 THEN
                SELECT IO_MAPPED_IO
            ELSE
                SELECT MEMORY_MAPPED_IO
            END IF

ELSE

IF AVAILABLE_MEMORY_SPACE >= REQUIRED_ADDRESS_SPACE THEN
                SELECT MEMORY_MAPPED_IO
            ELSE
                SELECT IO_MAPPED_IO
            END IF

END IF

END IF

CONFIGURE selected I/O technique

IF selected technique = MEMORY_MAPPED_IO THEN
        ASSIGN device address in memory address space
        USE normal memory read/write operations
    ELSE
        ASSIGN device address in I/O address space
        USE dedicated I/O read/write operations
    END IF

RETURN selected technique

END

Dynamic I/O Selection

START
  ↓
Read Latency + Bandwidth Requirements
  ↓
Check Address-Space Availability
  ↓
Is Very Low Latency Required?
 ┌──────────────┐
YES ↓           NO ↓
Check Memory     Check Bandwidth
Space             ↓
  ↓          High Bandwidth?
Available?    ┌────┴────┐
 ┌──┴──┐     YES       NO
YES   NO       ↓         ↓
 ↓     ↓    Check Fast   Check
Memory I/O  Memory Access Transfer Size
             ↓             ↓
           Select      Small → I/O-Mapped
         Appropriate   Large → Memory-Mapped
         Technique          ↓
              └─────────────┘
                    ↓
              Configure I/O
                    ↓
                   END
5. Decision Logic

The algorithm first examines the latency requirement. If extremely low latency is required, memory-mapped I/O can be selected when sufficient memory address space is available. This provides a convenient way for the processor to access device registers using its normal load and store mechanisms.

The algorithm then considers bandwidth. High-bandwidth devices may require efficient access mechanisms and can be configured using memory-mapped access when the processor and system architecture support it.

For smaller or conventional transfers, I/O-mapped I/O may be selected when a separate I/O address space is available. This avoids consuming part of the memory address space for device registers.

6. Memory-Mapped I/O

In memory-mapped I/O, device registers appear as memory addresses. For example:

DEVICE_STATUS ← MEMORY[ADDRESS]
MEMORY[DEVICE_DATA_ADDRESS] ← DATA

Advantages include:

• Same instructions can access memory and I/O.
• Large addressable space may be available depending on architecture.
• Convenient for processors with strong load/store operations.
• Device registers can be accessed using normal memory operations.

A limitation is that part of the memory address space is allocated to I/O devices.

7. I/O-Mapped I/O

In I/O-mapped I/O, a separate address space is provided for I/O devices. The processor uses dedicated instructions or mechanisms for accessing the device.

Example:

READ_IO DEVICE_PORT
WRITE_IO DEVICE_PORT, DATA

Advantages include:

• Does not consume normal memory address space.
• Provides a separate addressing mechanism.
• Can make the distinction between memory and I/O explicit.

8. Dynamic Selection

Dynamic selection means that the system does not permanently use one technique without considering application requirements. At configuration time, the system measures or receives the required latency and bandwidth and then chooses an appropriate technique.

A simplified decision rule is:

IF latency is very high priority AND memory space is available
    SELECT MEMORY-MAPPED I/O
ELSE IF I/O space is available AND transfer requirement is moderate
    SELECT I/O-MAPPED I/O
ELSE
    SELECT the technique supported by the processor/device
END IF

9. Advantages of the Algorithm

• Adapts to different I/O requirements.
• Considers latency and bandwidth.
• Prevents unnecessary consumption of address space.
• Provides a systematic selection process.
• Can be extended with additional device parameters.

10. Conclusion

The pseudocode dynamically selects between memory-mapped and I/O-mapped I/O according to latency, bandwidth, transfer size, address-space availability, and device capabilities. This provides a flexible approach to I/O communication and allows the system designer to select the most suitable addressing technique for a particular device or workload.

Q4. Design pseudocode for a bus arbitration algorithm that manages multiple devices communicating over PCIe, USB, and Thunderbolt interfaces.

1. Introduction

Bus arbitration is the process of deciding which device or communication request should receive access to a shared communication resource when multiple requests occur. Modern systems can contain several high-speed interfaces such as PCIe, USB, and Thunderbolt. A practical arbitration algorithm must consider priority, fairness, transfer size, interface status, and pending requests.

Although these interfaces use different physical and protocol mechanisms, an abstract arbitration algorithm can manage requests at the system level by maintaining queues and selecting the most suitable pending request.

2. Inputs

• Requests from PCIe devices
• Requests from USB devices
• Requests from Thunderbolt devices
• Device priority
• Transfer size
• Interface availability
• Queue status

3. Outputs

• Selected device/interface
• Granted communication opportunity
• Updated request queue
• Completion or error status

4. Pseudocode

BEGIN

INITIALIZE PCIe_QUEUE
    INITIALIZE USB_QUEUE
    INITIALIZE THUNDERBOLT_QUEUE

INITIALIZE DEVICE_PRIORITIES

WHILE SYSTEM_RUNNING = TRUE

COLLECT new requests from PCIe
        COLLECT new requests from USB
        COLLECT new requests from Thunderbolt

REMOVE completed requests

IF PCIe_QUEUE is not empty THEN
            ADD PCIe requests to PENDING_QUEUE
        END IF

IF USB_QUEUE is not empty THEN
            ADD USB requests to PENDING_QUEUE
        END IF

IF THUNDERBOLT_QUEUE is not empty THEN
            ADD Thunderbolt requests to PENDING_QUEUE
        END IF

IF PENDING_QUEUE is empty THEN
            WAIT for new request
            CONTINUE
        END IF

FOR each request in PENDING_QUEUE
            CALCULATE REQUEST_SCORE
        END FOR

SELECT request with highest REQUEST_SCORE

IF selected interface is AVAILABLE THEN

GRANT selected request

PERFORM data transfer

IF transfer completed successfully THEN
                MARK request COMPLETE
                REMOVE request from PENDING_QUEUE
            ELSE
                MARK request ERROR
                RETRY or REPORT error
            END IF

RELEASE communication resource

ELSE
            KEEP request in queue
        END IF

UPDATE fairness counters
        UPDATE priorities if required

END WHILE

END

PCIe / USB / Thunderbolt Arbitration

START
  ↓
Initialize Interface Queues
  ↓
Collect PCIe, USB and Thunderbolt Requests
  ↓
Build Pending Queue
  ↓
Any Pending Request?
 ┌──────────────┐
NO ↓            YES ↓
Wait          Calculate Request Scores
                  ↓
             Select Highest Suitable
                  ↓
             Interface Available?
             ┌────┴────┐
            NO         YES
            ↓           ↓
      Keep in Queue   Grant Request
                         ↓
                    Transfer Data
                         ↓
                   Transfer Complete?
                    ┌────┴────┐
                   NO        YES
                   ↓          ↓
               Retry/Error  Mark Complete
                              ↓
                       Update Fairness
                              ↓
                             END

5. Request Score

A score can be calculated using:

REQUEST_SCORE =
    DEVICE_PRIORITY
    + WAIT_TIME_WEIGHT
    + URGENCY_WEIGHT
    - TRANSFER_COST

The exact weights can be changed according to system requirements.

A high-priority request receives a larger score. A request that has waited for a long time also receives an increased score. This prevents low-priority devices from waiting indefinitely.

6. Fairness

If only priority is used, a continuous stream of high-priority requests could prevent lower-priority requests from receiving service. To avoid starvation, the algorithm increases the priority of waiting requests.

Pseudocode:

FOR each waiting request

WAIT_TIME ← WAIT_TIME + 1

IF WAIT_TIME > STARVATION_LIMIT THEN
        PRIORITY ← PRIORITY + AGING_VALUE
    END IF

END FOR

This technique is called priority aging.

7. PCIe Handling

For PCIe requests, the arbitration layer checks whether the PCIe path is available and whether the requesting device is ready for the transaction. Once selected, the transaction is allowed to proceed according to the interface's normal protocol.

8. USB Handling

USB requests are placed into the USB request queue. The arbitration algorithm considers the priority and waiting time of each request. Once a USB request is selected, it is granted access according to the system's USB scheduling and communication rules.

9. Thunderbolt Handling

Thunderbolt requests are similarly placed into a dedicated queue. The algorithm checks link availability and selects a request based on priority, waiting time, and transfer requirements.

10. Error Handling

IF INTERFACE_ERROR = TRUE THEN

RECORD error
    RELEASE interface
    RETRY request if retry is allowed

IF retry_count > MAX_RETRIES THEN
        REPORT FAILURE
        REMOVE request
    END IF

END IF

11. Advantages

• Handles requests from multiple interfaces.
• Uses priority-based scheduling.
• Provides fairness through aging.
• Supports high-speed data transfers.
• Maintains separate interface queues.
• Includes error handling.
• Can be adapted for different workloads.

12. Conclusion

The proposed bus arbitration algorithm collects requests from PCIe, USB, and Thunderbolt interfaces and places them into queues. It calculates a request score using priority, waiting time, urgency, and transfer cost. The highest suitable request is granted communication access. Priority aging prevents starvation and provides fairness among devices. Thus, the algorithm provides an organized approach for managing multiple communication requests.

Q5. Write pseudocode to implement a hybrid I/O communication mechanism that uses polling for low-speed devices and DMA with interrupts for high-speed devices.

1. Introduction

Different I/O devices have different performance requirements. Low-speed devices generally transfer small amounts of data and may not justify the overhead associated with DMA configuration and interrupt processing. High-speed devices, on the other hand, can generate large amounts of data and require efficient transfer mechanisms.

A hybrid I/O mechanism combines polling and DMA with interrupts. Polling is used for low-speed devices because it is simple and suitable for occasional small transfers. DMA with interrupts is used for high-speed devices because it reduces CPU involvement and efficiently handles large blocks of data.

2. Inputs

• Device type
• Device speed
• Data size
• Required bandwidth
• Device status
• DMA controller availability
• Interrupt availability

3. Outputs

• Selected I/O method
• Completed data transfer
• Device status
• Error or completion notification

4. Pseudocode

BEGIN

INITIALIZE I/O SYSTEM
    INITIALIZE DMA CONTROLLER
    INITIALIZE INTERRUPT CONTROLLER

FOR each connected DEVICE

READ DEVICE_SPEED
        READ TRANSFER_SIZE
        READ DEVICE_STATUS

IF DEVICE_SPEED = LOW THEN

SELECT POLLING_MODE

WHILE DATA_AVAILABLE = FALSE
                CHECK DEVICE_STATUS
            END WHILE

READ data from device

STORE data in memory

CLEAR device status

ELSE IF DEVICE_SPEED = HIGH THEN

SELECT DMA_MODE

CONFIGURE DMA CONTROLLER
            SET SOURCE = DEVICE
            SET DESTINATION = MEMORY
            SET TRANSFER_SIZE = required size
            ENABLE DMA INTERRUPT

START DMA TRANSFER

CPU CONTINUES OTHER TASKS

WAIT FOR DMA INTERRUPT

WHEN DMA_INTERRUPT_RECEIVED DO

READ DMA_STATUS

IF DMA_STATUS = COMPLETE THEN
                    CLEAR DMA INTERRUPT
                    RECORD successful transfer
                ELSE
                    HANDLE DMA ERROR
                END IF

END WHEN

ELSE

IF TRANSFER_SIZE <= SMALL_TRANSFER_LIMIT THEN
                SELECT POLLING_MODE
            ELSE
                SELECT DMA_MODE
            END IF

END IF

END FOR

END

– Hybrid I/O

START
  ↓
Detect Device Speed + Transfer Size
  ↓
Is Device Low-Speed?
 ┌──────────────┐
YES ↓           NO ↓
Select Polling   Select DMA + Interrupt
     ↓                    ↓
Check Device Status   Configure DMA
     ↓                    ↓
Data Available?       Start DMA
 ┌───┴───┐              ↓
NO      YES        CPU Performs Other Work
 ↓        ↓              ↓
Poll    Read Data    DMA Interrupt?
                      ┌───┴────┐
                     NO       YES
                     ↓         ↓
                   Continue  Run ISR
                               ↓
                          Check Status
                               ↓
                              END

5. Polling Method for Low-Speed Devices

Polling means that the CPU repeatedly checks the status of an I/O device until the device becomes ready. It is simple and does not require interrupt configuration.

Example:

WHILE DEVICE_READY = FALSE
    CHECK DEVICE_STATUS
END WHILE

READ DEVICE_DATA

For low-speed devices, the CPU may not lose a significant amount of processing capacity because data arrives infrequently. Therefore, polling can be an appropriate solution.

6. DMA with Interrupts for High-Speed Devices

High-speed devices can transfer large amounts of data. If the CPU were responsible for every transfer, processor overhead would become high. DMA solves this problem by allowing the DMA controller to transfer blocks of data directly between the device and memory.

The CPU configures the DMA controller and then continues with other tasks. When the DMA transfer is complete, an interrupt is generated. The CPU executes the corresponding ISR.

7. Dynamic Decision

The selection can be represented by:

IF DEVICE_SPEED <= LOW_SPEED_LIMIT THEN
    USE POLLING
ELSE
    USE DMA + INTERRUPT
END IF

An improved decision can also include transfer size:

IF DEVICE_SPEED = LOW AND TRANSFER_SIZE < SMALL_LIMIT THEN
    USE POLLING
ELSE IF DEVICE_SPEED = HIGH OR TRANSFER_SIZE >= LARGE_LIMIT THEN
    USE DMA + INTERRUPT
ELSE
    SELECT based on CPU load and device requirements
END IF

8. CPU Involvement

In polling mode, CPU involvement is continuous while waiting for the device.

In DMA mode, CPU involvement is limited mainly to:

• Configuring DMA.
• Starting the transfer.
• Handling completion interrupts.
• Handling errors.

Therefore, the hybrid system uses CPU resources according to the needs of each device.

9. Error Handling

IF POLLING_MODE AND DEVICE_ERROR = TRUE THEN
    RECORD device error
    RESET device if possible
END IF

IF DMA_MODE AND DMA_ERROR = TRUE THEN
    STOP DMA
    RECORD error
    CLEAR interrupt
    RETRY if permitted
END IF

10. Example

Consider a system containing a keyboard and a high-speed storage device.

The keyboard is a low-speed device. It produces small amounts of data occasionally. Polling can be used to check whether a key is available.

The high-speed storage device can transfer a large block of data. DMA can transfer the block directly into memory, while an interrupt informs the CPU after completion.

Thus:

Keyboard → Polling → CPU reads small data
High-speed device → DMA → Memory → Interrupt → CPU

11. Advantages

• Uses simple polling for low-speed devices.
• Uses efficient DMA for high-speed devices.
• Reduces CPU overhead for large transfers.
• Uses interrupts only when beneficial.
• Provides flexibility.
• Improves overall I/O performance.
• Supports different device characteristics in the same system.

12. Conclusion

The hybrid I/O mechanism selects the communication technique according to device speed and transfer requirements. Low-speed devices use polling because their transfers are small and infrequent. High-speed devices use DMA with interrupts because large data transfers can occur without continuous CPU intervention. This combination provides a balanced and efficient I/O communication strategy.