SIMATS ENGINEERING

Department of Computer Science & Engineering

ASSESSMENT TOOL 2 - COMPARATIVE ANALYSIS

| Course Code: | CSA12 | Course Title: | Computer Architecture |
| --- | --- | --- | --- |
| CO Assessed: | CO4 | Assessment: | ASSESSMENT TOOL 2-COMPARATIVE ANALYSIS |
| Weightage: | 40% | Total Marks: | 25 |
| CO4 Statement: | Implement hierarchical memory systems by differentiating cache levels (L1, L2, L3), virtual memory with paging, and advanced memory technologies (DRAM, SRAM, NAND Flash, MRAM, RRAM) based on latency, bandwidth, and throughput characteristics. |  |  |
| Student Name: |  | Reg.No.: |  |
| Department: |  | Date: |  |

ANNEXURE A

COMPARATIVE ANALYSIS

1.Compare L1, L2, and L3 cache levels in terms of latency, bandwidth, and throughput, and analyze their impact on processor performance.

2.Differentiate SRAM and DRAM by evaluating their latency, bandwidth, and throughput characteristics in cache and main memory design.

3.Compare virtual memory with paging and cache memory mechanisms in terms of access latency and system throughput.

4.Analyze the differences between NAND Flash and DRAM with respect to latency, bandwidth, and suitability for hierarchical memory systems.

5.Compare MRAM and RRAM technologies by examining their performance in terms of access time, throughput, and energy efficiency.

Code of Conduct:

I DEEPTHI KORRAPATI(192572012) (Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student:

MARKS ALLOCATION

|  | Scope of Items(20%) | Comparison Depth(25%) | Use of Evidence(20%) | Critical Thinking(20%) | Clarity of Presentation (15%) |
| --- | --- | --- | --- | --- | --- |
| Q1 |  |  |  |  |  |
| Q2 |  |  |  |  |  |
| Q3 |  |  |  |  |  |
| Q4 |  |  |  |  |  |
| Q5 |  |  |  |  |  |

RUBRICS FOR CALCULATION:

| Criteria | Weightage | Level 4 – Exemplary | Level 3 – Proficient | Level 2 – Developing | Level 1 – Beginning |
| --- | --- | --- | --- | --- | --- |
| Scope of Items | 20% | Selects highly relevant items with clear scope and justification | Selects appropriate items with minor gaps | Selection is limited or partially relevant | Items are unclear or not relevant |
| Comparison Depth | 25% | Provides detailed and comprehensive comparison across multiple aspects | Provides adequate comparison with some depth | Limited comparison with few aspects | Very minimal or no comparison |
| Use of Evidence | 20% | Supports comparison with accurate data, examples, or references | Uses some supporting evidence with minor gaps | Limited or weak supporting evidence | No supporting evidence provided |
| Critical Thinking | 20% | Demonstrates strong critical thinking with clear interpretation and reasoning | Shows reasonable interpretation with minor gaps | Basic interpretation; lacks depth | No meaningful interpretation |
| Clarity of Presentation | 15% | Well-organized, clear, and logically structured presentation | Generally clear with minor issues | Somewhat unclear or poorly structured | Disorganized and difficult to understand |

QUESTION 1

Compare L1, L2, and L3 cache levels in terms of latency, bandwidth, and throughput, and analyze their impact on processor performance.

ANSWER

1. Introduction

Cache memory is a high-speed memory located close to the processor. The main purpose of cache is to reduce the time required by the CPU to access frequently used instructions and data. A modern processor generally uses multiple levels of cache, namely L1, L2, and L3.

These three cache levels form a hierarchy. L1 is the smallest and fastest, L2 is larger but slightly slower, and L3 is usually the largest and slowest among the three. The processor first searches the closest cache. If the required data is not found, it moves to the next level.

The basic hierarchy is:

CPU → L1 Cache → L2 Cache → L3 Cache → Main Memory (DRAM)

The use of multiple cache levels allows the processor to achieve a balance between speed, capacity, cost, bandwidth, and power consumption.

2. L1 Cache

L1 cache is the fastest cache level.

It is located very close to the CPU core and normally has a small capacity.

L1 cache is commonly divided into:

- Instruction Cache (L1-I)

- Data Cache (L1-D)

The instruction cache stores frequently required instructions, while the data cache stores frequently accessed data.

The major advantage of L1 cache is its extremely low access latency. Since it is located close to the processor, the CPU can access it very quickly.

However, because L1 must be extremely fast, its capacity is limited.

3. L2 Cache

L2 cache is the second level of cache.

It is larger than L1 but has slightly higher latency.

If the required data is not available in L1, the processor checks L2.

For example:

CPU → L1 Miss → L2 Hit

In this situation, the processor obtains the required data from L2 without accessing L3 or DRAM.

L2 therefore reduces the performance penalty caused by L1 cache misses.

4. L3 Cache

L3 cache is generally larger than L1 and L2.

It is often shared between multiple processor cores.

Its main purpose is to reduce the number of accesses to DRAM.

For example:

CPU → L1 Miss → L2 Miss → L3 Hit

The processor can still obtain the required data from cache without accessing slower main memory.

L3 has higher latency than L1 and L2 but provides greater capacity.

5. Latency Comparison

Latency is the amount of time required to access data.

The general relationship is:

L1 latency < L2 latency < L3 latency < DRAM latency

Therefore:

- L1 provides the lowest latency.

- L2 provides low latency with greater capacity.

- L3 provides higher capacity at somewhat higher latency.

- DRAM is slower but provides much greater capacity.

Low latency is particularly important for CPU-intensive applications.

6. Bandwidth

Bandwidth represents the amount of data that can be transferred within a given period.

Cache memory can provide very high effective bandwidth because it is located close to the processor.

L1 generally provides the highest bandwidth among cache levels.

L2 provides high bandwidth but is slower than L1.

L3 provides substantial bandwidth but normally has higher access latency.

High cache bandwidth allows the CPU to receive data quickly and reduces processor waiting time.

7. Throughput

Throughput refers to the amount of useful processing or data transfer completed per unit time.

An effective cache hierarchy improves processor throughput because the CPU spends less time waiting for data.

For example, if most memory requests are satisfied by L1 or L2, the processor can continue executing instructions without frequently waiting for DRAM.

Therefore:

Higher cache hit rate → fewer memory stalls → higher CPU throughput

8. Comparison Table

| Feature | L1 Cache | L2 Cache | L3 Cache |
| --- | --- | --- | --- |
| Size | Smallest | Medium | Largest |
| Latency | Lowest | Low | Higher |
| Bandwidth | Very high | High | High |
| Cost per bit | High | High | Lower than L1 |
| Location | Closest to CPU core | Close to CPU | Usually shared |
| Main purpose | Critical data/instructions | Reduce L1 misses | Reduce DRAM accesses |

9. Cache Hit and Cache Miss

A cache hit occurs when the required data is found in the cache.

A cache miss occurs when the required data is not found.

For example:

CPU → L1 Hit

The processor obtains data immediately from L1.

But:

CPU → L1 Miss → L2 Miss → L3 Miss → DRAM

requires progressively slower memory accesses.

Therefore, improving cache hit rates is an important way to improve processor performance.

10. Impact on Processor Performance

A good cache hierarchy improves:

- Instruction execution speed

- Application responsiveness

- Throughput

- CPU utilization

- Energy efficiency

If the processor frequently accesses DRAM, the CPU may remain idle while waiting for memory.

A well-designed cache hierarchy minimizes these stalls.

11. Locality of Reference

Cache memory works because programs usually exhibit locality.

Temporal Locality

Recently accessed data is likely to be accessed again.

Spatial Locality

Data located near recently accessed data is likely to be accessed soon.

Caches exploit both forms of locality.

12. Overall Analysis

L1 provides maximum speed but minimum capacity.

L2 provides a balance between speed and capacity.

L3 provides large cache capacity and reduces DRAM traffic.

Thus, the three levels complement each other rather than replacing one another.

13. Conclusion

L1, L2 and L3 caches are essential components of modern processor architecture.

The hierarchy can be summarized as:

L1 → Fastest and smallest
L2 → Larger and slightly slower
L3 → Largest cache and slower than L1/L2

L1 minimizes latency for critical data, L2 reduces the penalty of L1 misses, and L3 reduces expensive DRAM accesses.

Therefore, an efficient cache hierarchy increases processor throughput, reduces memory stalls and improves overall system performance.

QUESTION 2

Differentiate SRAM and DRAM by evaluating their latency, bandwidth, and throughput characteristics for cache and main memory design.

ANSWER

1. Introduction

SRAM and DRAM are two important types of semiconductor memory used in computer systems.

SRAM stands for Static Random Access Memory, while DRAM stands for Dynamic Random Access Memory.

Both are volatile memories, meaning their stored information is lost when power is removed. However, their internal construction and performance characteristics are different.

SRAM is mainly used for cache memory, while DRAM is mainly used for main memory.

2. SRAM

SRAM stores each bit using a circuit based on multiple transistors.

It does not require the same periodic refresh operation as DRAM.

The major advantage of SRAM is its high speed.

It provides:

- Very low latency

- High performance

- High access speed

- High bandwidth

However, SRAM requires more physical area per bit.

Therefore, it is expensive and unsuitable for very large-capacity main memory.

3. DRAM

DRAM stores information using a capacitor-based memory cell.

Because the stored charge gradually leaks away, DRAM requires periodic refreshing.

The refresh requirement contributes to its complexity and performance characteristics.

However, DRAM has a much higher density than SRAM.

Therefore, it is suitable for large-capacity main memory.

4. Latency Comparison

SRAM has lower access latency than DRAM.

This makes it suitable for cache memory.

The processor needs extremely fast access to frequently used instructions and data.

Therefore:

CPU → SRAM Cache

is highly effective for reducing processor waiting time.

DRAM has higher latency but provides much greater capacity.

Therefore, it is used as the main memory.

5. Bandwidth

Bandwidth represents how much data can be transferred in a given period.

SRAM can provide very high effective bandwidth because of its fast access characteristics.

DRAM can also provide high bandwidth, particularly in modern systems using multiple memory channels.

However, the key difference is that SRAM provides speed with small capacity, while DRAM provides large capacity with somewhat higher latency.

6. Throughput

Throughput refers to the amount of useful work completed in a particular period.

SRAM improves processor throughput because the CPU can access frequently used information quickly.

DRAM supports overall system throughput by providing enough capacity for large applications and datasets.

Thus, both memories contribute to throughput in different ways.

7. Cache Memory Design

SRAM is ideal for cache memory because cache requires:

- Low latency

- High speed

- High bandwidth

- Frequent access

The processor can access frequently used data without going to DRAM.

A typical hierarchy is:

CPU → SRAM Cache → DRAM

8. Main Memory Design

DRAM is suitable for main memory because it provides:

- High density

- Large capacity

- Lower cost per bit

- Good bandwidth

Applications, operating-system data and active datasets can be stored in DRAM.

9. Comparison Table

| Characteristic | SRAM | DRAM |
| --- | --- | --- |
| Full form | Static RAM | Dynamic RAM |
| Latency | Very low | Higher |
| Speed | Very high | High |
| Refresh | Not required like DRAM | Required |
| Density | Lower | Higher |
| Cost per bit | High | Lower |
| Capacity | Smaller | Larger |
| Typical use | Cache | Main memory |
| Bandwidth | Very high | High |
| Power considerations | Suitable for small fast memories | Efficient for large capacities |

10. Why SRAM Is Used for Cache

Cache memory is accessed extremely frequently.

If cache were made using slower memory, the CPU would spend more time waiting.

SRAM provides the low latency required by L1, L2 and portions of other cache structures.

However, creating a large cache entirely from SRAM would increase:

- Chip area

- Manufacturing cost

- Power consumption

Therefore, SRAM is used in relatively smaller quantities.

11. Why DRAM Is Used for Main Memory

A computer needs much more main memory than cache.

For example, an application may require gigabytes of working memory.

Creating such a large memory using SRAM would be expensive and inefficient.

DRAM provides high density and therefore makes large-capacity memory practical.

12. Latency-Bandwidth Trade-Off

The memory hierarchy represents a trade-off.

SRAM:

Low latency + high speed + low capacity

DRAM:

Higher latency + high capacity + good bandwidth

This trade-off allows computer architects to combine both technologies.

13. Impact on System Performance

If the processor finds frequently used data in SRAM cache, it can execute instructions rapidly.

If the data is not available in cache, the processor must access DRAM.

Therefore:

Higher cache hit rate → fewer DRAM accesses → lower effective latency

This improves processor performance.

14. Example

Consider a processor repeatedly accessing the same data.

If the data is stored in SRAM cache:

CPU → Cache Hit → Data

The access is very fast.

If the data is not present:

CPU → Cache Miss → DRAM → Data

The processor experiences additional latency.

15. Conclusion

SRAM and DRAM are both essential to hierarchical memory systems.

SRAM has very low latency and high speed, making it ideal for processor cache.

DRAM has greater capacity and higher density, making it ideal for main memory.

The recommended design is:

CPU → SRAM Cache → DRAM

This combination provides low latency for frequently used data while maintaining sufficient capacity for large applications and datasets.

QUESTION 3

Compare virtual memory with paging and cache memory mechanisms in terms of access latency and system throughput.

ANSWER

1. Introduction

Modern computer systems use several memory-management mechanisms to improve performance and efficiently manage limited physical memory.

Two important mechanisms are:

- Cache memory

- Virtual memory with paging

Although both are related to memory access, they have different purposes.

Cache memory mainly reduces processor memory-access latency.

Virtual memory allows applications to use a larger logical address space and provides memory management and isolation.

2. Cache Memory

Cache is a small and fast memory located close to the processor.

The common hierarchy is:

L1 → L2 → L3 → DRAM

Cache stores frequently accessed data and instructions.

It relies on locality of reference.

3. Cache Hit

A cache hit occurs when requested data is found in the cache.

Example:

CPU → L1 Cache → Data Found

The processor receives data quickly.

This improves execution speed and throughput.

4. Cache Miss

A cache miss occurs when data is not available in the required cache level.

The processor searches lower levels.

Example:

L1 Miss → L2 Miss → L3 Miss → DRAM

This increases access latency.

5. Virtual Memory

Virtual memory provides an abstraction in which applications operate using virtual addresses.

The operating system maps virtual addresses to physical memory.

This allows processes to have their own address spaces.

6. Paging

Paging divides memory into fixed-size units called pages.

Physical memory is divided into corresponding frames.

The operating system maintains mappings between virtual pages and physical frames.

When a required page is available in physical memory, it can be accessed normally.

7. Page Fault

A page fault occurs when the required virtual page is not currently available in physical memory.

The operating system may need to retrieve the page from secondary storage.

This is much slower than cache or DRAM access.

Therefore:

Page fault → Very high latency

8. Comparison

| Feature | Cache Memory | Virtual Memory with Paging |
| --- | --- | --- |
| Main purpose | Reduce CPU access latency | Manage memory/address space |
| Managed by | Primarily hardware | OS + hardware |
| Storage unit | Cache line/block | Page |
| Speed | Very high | Page faults are very slow |
| Main benefit | Faster execution | Larger logical address space |
| Main problem | Cache miss | Page fault |
| Effect on throughput | Usually improves it | Excessive paging can reduce it |

9. Access Latency

Cache has extremely low latency compared with DRAM.

Virtual memory itself does not necessarily mean every access is slow. If the required page is already in DRAM, normal memory access occurs.

However, a page fault can cause a major delay because storage access is much slower.

Therefore:

Cache hit → Very low latency

DRAM access → Higher latency

Page fault → Extremely high latency

10. Impact on Throughput

Cache improves throughput by reducing CPU stalls.

Virtual memory improves system flexibility but excessive paging can reduce throughput.

If the system continuously moves pages between RAM and storage, it may enter a condition of heavy paging activity.

This can consume system resources and reduce useful application execution.

11. Role of Locality

Cache depends heavily on locality.

If an application repeatedly accesses the same data, cache hit rates increase.

Paging also benefits when applications have reasonable locality because active pages can remain in physical memory.

12. Recommended Hierarchy

A practical hierarchy is:

CPU → L1 → L2 → L3 → DRAM → Storage

Cache handles fast access.

DRAM stores active pages.

Storage acts as a slower backing layer for virtual memory.

13. Improving Performance

System performance can be improved by:

- Increasing cache effectiveness

- Improving cache hit rates

- Providing sufficient DRAM

- Reducing unnecessary paging

- Improving memory locality

- Optimizing application memory usage

14. Example

Suppose a program repeatedly accesses a small dataset.

With good cache locality:

CPU → L1/L2 → Data

The processor executes efficiently.

If the system does not have enough physical memory:

CPU → DRAM → Page Fault → Storage

This creates significant latency.

15. Conclusion

Cache memory and virtual memory with paging solve different problems.

Cache reduces processor access latency by keeping frequently used information close to the CPU.

Virtual memory provides address-space management and allows pages to be moved between physical memory and secondary storage.

The best system uses both:

Cache → reduce latency

DRAM → provide fast working memory

Paging → manage memory capacity

Excessive cache misses reduce performance, while excessive page faults can severely reduce throughput.

Therefore, sufficient DRAM, effective cache utilization and controlled paging are essential for high-performance systems.

QUESTION 4

Analyze the differences between NAND Flash and DRAM with respect to latency, bandwidth, and suitability for hierarchical memory systems.

ANSWER

1. Introduction

NAND Flash and DRAM are both widely used memory technologies, but they have very different characteristics.

DRAM is primarily used as main memory, while NAND Flash is mainly used for persistent storage.

The main difference is that DRAM is volatile and fast, whereas NAND Flash is non-volatile and provides high-density storage.

2. NAND Flash

NAND Flash is a type of non-volatile memory.

It retains stored information even when power is removed.

It is commonly used in:

- SSDs

- Smartphones

- Memory cards

- USB storage

- Embedded systems

Its major advantage is high storage density.

3. DRAM

DRAM is volatile memory.

It is commonly used as the main memory of computers and other processing systems.

It provides much faster access than NAND Flash.

It is suitable for actively running applications.

4. Latency

DRAM has significantly lower access latency than NAND Flash.

This makes DRAM suitable for active processing.

NAND Flash has much higher access latency and is therefore not normally used directly as processor main memory.

The hierarchy can therefore be represented as:

CPU → Cache → DRAM → NAND Flash

5. Bandwidth

DRAM provides high memory bandwidth.

This is important because processors and accelerators may continuously transfer large quantities of data.

NAND Flash can provide substantial storage throughput, particularly in parallel flash-based systems, but its access behavior is different from DRAM.

Therefore, DRAM remains more suitable for active working memory.

6. Volatility

DRAM is volatile.

If power is removed, stored information is lost.

NAND Flash is non-volatile.

Data remains stored without continuous power.

This makes NAND Flash suitable for long-term data storage.

7. Capacity

NAND Flash provides high storage density.

Large amounts of information can be stored in relatively compact devices.

DRAM is generally more expensive per bit, so it is used in smaller quantities compared with mass storage.

8. Comparison Table

| Feature | DRAM | NAND Flash |
| --- | --- | --- |
| Volatility | Volatile | Non-volatile |
| Latency | Low | Much higher |
| Bandwidth | High | High storage throughput, but different access characteristics |
| Capacity | Moderate/high | Very high |
| Main purpose | Working memory | Persistent storage |
| Power | Requires active operation/refresh | Retains data without power |
| Typical location | Main memory | SSD/storage |
| Processor use | Direct working memory | Usually accessed through storage interface |

9. Role in Hierarchical Memory

DRAM should be positioned closer to the processor.

NAND Flash should be positioned below DRAM.

A simplified hierarchy is:

Registers
↓
L1 Cache
↓
L2 Cache
↓
L3 Cache
↓
DRAM
↓
NAND Flash

As we move downward:

- Capacity increases

- Cost per bit decreases

- Latency generally increases

10. Why DRAM Is Suitable for Main Memory

Applications require rapid access to their active data.

DRAM can provide the required speed and bandwidth.

For example, when a program is executing:

CPU → Cache → DRAM

The processor can access active program data relatively quickly.

11. Why NAND Flash Is Suitable for Storage

NAND Flash provides large persistent capacity.

It can store:

- Operating systems

- Applications

- Documents

- Videos

- Games

- AI models

When data is required for active execution, it can be transferred from NAND Flash into DRAM.

12. Hierarchical Operation

Consider launching an application.

Initially:

Application stored in NAND Flash

When launched:

NAND Flash → DRAM

During execution:

DRAM → Cache → CPU

This arrangement allows the system to combine the advantages of both technologies.

13. Latency-Bandwidth Trade-Off

DRAM provides low latency and high bandwidth for active computation.

NAND Flash provides high capacity and non-volatility but higher access latency.

Therefore, the technologies should not be viewed as direct replacements.

They are complementary.

14. Improving System Performance

A system can improve performance by:

- Providing sufficient DRAM capacity

- Using high-bandwidth DRAM

- Efficiently transferring data from NAND

- Caching frequently accessed storage data

- Using compression where appropriate

- Reducing unnecessary storage transfers

15. Conclusion

NAND Flash and DRAM have different roles in a hierarchical memory system.

DRAM is fast and suitable for active working memory.

NAND Flash is slower but provides large, non-volatile storage.

The recommended hierarchy is:

Cache → DRAM → NAND Flash

DRAM provides low-latency access and high bandwidth, while NAND Flash provides high-capacity persistent storage.

Using both technologies appropriately creates a balanced memory system with good performance, capacity and persistence.

\

QUESTION 5

Compare MRAM and RRAM technologies by examining their performance in terms of access time, throughput, and energy efficiency.

ANSWER

1. Introduction

Modern computer systems increasingly require memory technologies that provide high performance while consuming less energy.

Two emerging non-volatile memory technologies are:

- MRAM – Magnetoresistive Random Access Memory

- RRAM – Resistive Random Access Memory

Both technologies can retain data without continuous power, but they use different physical mechanisms.

They are being studied for applications ranging from embedded memory to advanced computing systems.

2. MRAM

MRAM stores information using magnetic states.

Because it is non-volatile, data can remain available even when power is removed.

Important characteristics of MRAM include:

- Non-volatility

- Fast access

- High endurance

- Potentially low standby power

- Suitability for embedded applications

3. RRAM

RRAM stores information using different resistance states in a resistive memory element.

The resistance state represents stored information.

RRAM is also non-volatile.

It has attracted attention because of its potential for:

- High density

- Low-power memory

- Embedded applications

- In-memory computing

- AI acceleration

4. Access Time

Access time refers to the time required to read or write information.

MRAM can provide relatively fast access and is attractive for applications requiring both speed and non-volatility.

RRAM can also provide fast access, although actual performance depends heavily on device design and implementation.

Therefore, both can occupy positions between conventional volatile memory and slower persistent storage depending on the architecture.

5. Throughput

Throughput represents the amount of useful data processed or transferred per unit time.

High throughput depends not only on the memory cell itself but also on:

- Memory architecture

- Number of memory banks

- Parallelism

- Controller design

- Read/write operations

MRAM can support high-performance memory operations.

RRAM can potentially provide significant throughput, particularly in architectures that exploit parallel resistive devices.

6. Energy Efficiency

Energy efficiency is an important advantage of non-volatile memory.

Since MRAM and RRAM retain information without continuous power, they can reduce standby energy requirements compared with some volatile memories.

MRAM can be attractive for low-power embedded systems because data does not need to be continuously refreshed like DRAM.

RRAM can also offer low-energy operation depending on the implementation.

7. Comparison Table

| Feature | MRAM | RRAM |
| --- | --- | --- |
| Full form | Magnetoresistive RAM | Resistive RAM |
| Storage principle | Magnetic state | Resistance state |
| Volatility | Non-volatile | Non-volatile |
| Access time | Fast | Potentially fast |
| Throughput | High potential | High potential with parallelism |
| Standby power | Low potential | Low potential |
| Density | Technology-dependent | Potentially high |
| Special advantage | Fast non-volatile memory | High density and in-memory computing potential |
| Applications | Embedded systems, persistent memory | AI, embedded systems, in-memory computing |

8. MRAM Performance

MRAM's non-volatility means that information can be retained without continuous power.

This can be useful in systems that frequently enter low-power states.

For example:

Power ON → Data available

after previously stored information has been retained.

This can reduce startup overhead in suitable systems.

9. RRAM Performance

RRAM is particularly interesting for future computing architectures.

One important possibility is in-memory computing.

Traditional computing often involves:

Memory → Transfer Data → Processor → Transfer Result → Memory

This repeated movement can consume energy.

An RRAM-based architecture may perform certain computational operations closer to where data is stored.

This can reduce data movement.

10. Energy Comparison

Energy efficiency depends on the particular implementation.

MRAM benefits from non-volatility and can reduce standby power.

RRAM may also provide low-energy operation and high density.

However, write energy, endurance, switching characteristics and peripheral circuitry must be considered when evaluating a real design.

11. Throughput and Parallelism

Both technologies can benefit from parallel memory organization.

For RRAM, arrays of resistive devices can potentially be used for parallel operations.

This is particularly useful for AI workloads where many multiply-accumulate-like operations may be performed.

MRAM can also be organized into high-performance memory arrays.

12. Applications

MRAM can be useful for:

- Embedded memory

- Automotive systems

- Industrial systems

- Persistent memory

- Low-power devices

RRAM can be useful for:

- AI accelerators

- Embedded memory

- Neuromorphic computing

- In-memory computing

- High-density storage applications

13. Important Trade-Offs

Neither technology is universally better.

The selection depends on:

- Required access time

- Read/write requirements

- Energy budget

- Density

- Endurance

- Manufacturing technology

- Application workload

For example, an embedded system requiring rapid persistent state may benefit from MRAM.

An AI accelerator focused on reducing data movement may benefit from an RRAM-based architecture.

14. Overall Comparison

In simple terms:

MRAM:

Fast + non-volatile + good endurance + low standby power potential.

RRAM:

Non-volatile + potentially high density + low-energy operation + strong potential for in-memory computing.

Both technologies can complement conventional memory systems.

15. Conclusion

MRAM and RRAM are promising non-volatile memory technologies with different strengths.

MRAM provides fast access, non-volatility and strong potential for low-power embedded applications.

RRAM provides non-volatility, potentially high density and opportunities for highly parallel or in-memory computing architectures.

The final choice should depend on the application.

For general fast non-volatile embedded memory, MRAM can be highly suitable.

For AI and architectures where reducing data movement is important, RRAM can provide significant potential.

Thus, both technologies are important candidates for future hierarchical memory systems.