SIMATS ENGINEERING

Department of Computer Science & Engineering

ASSESSMENT TOOL 1-CONSTRAINT-BASED PROBLEM SOLVING

| Course Code: | CSA12 | Course Title: | Computer Architecture |
| --- | --- | --- | --- |
| CO Assessed: | CO5 | Assessment: | ASSESSMENT TOOL1-Constraint-Based Problem Solving |
| Weightage: | 60% | Total Marks: | 25 |
| CO5Statement: | Construct I/O communication mechanisms by comparing memory-mapped and I/O-mapped techniques, interrupt handling (vectored, hardware, software), DMA controllers, and advanced bus interfaces including PCIe, USB, NVMe, and Thunderbolt. |  |  |
| Student Name: |  | Reg.No.: |  |
| Department: |  | Date: |  |

ANNEXURE A

Constraint-Based Problem Solving

1.Construct an I/O communication mechanism for a real-time embedded system with the following constraints:

- CPU utilization must be below 15%

- Multiple sensors generate interrupts every 1 ms

- Use vectored interrupt handling

- Select between memory-mapped and I/O-mapped I/O for optimal latency

2.Design an I/O subsystem under these constraints:

- High-speed storage requires throughput > 3 GB/s

- Must use NVMe over PCIe

- CPU should not handle bulk data transfer

- Integrate DMA controller with interrupt notification

3.Construct a multi-device communication architecture with constraints:

- 6 peripherals using USB, 2 high-speed devices using Thunderbolt

- Avoid interrupt starvation

- Use hardware and software interrupts

- Ensure fair bus arbitration

4.Design a data acquisition system with constraints:

- Continuous data stream at 500 MB/s

- Minimal CPU intervention required

- Must compare DMA transfer vs interrupt-driven I/O

- Use memory-mapped I/O for device registers

5.Construct an interrupt handling mechanism with constraints:

- Support nested interrupts

- Maximum interrupt latency < 10 µs

- Use vectored interrupts for high-priority devices

- Use software interrupts for background tasks

Code of Conduct:

I DEEPTHI KORRAPATI(192572012) (Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student:

MARKS ALLOCATION

|  | Problem Understanding(25%) | Constraint Analysis(25%) | Solution Development(25%) | Application of Concepts(15%) | Justification(10%) |
| --- | --- | --- | --- | --- | --- |
| Q1 |  |  |  |  |  |
| Q2 |  |  |  |  |  |
| Q3 |  |  |  |  |  |
| Q4 |  |  |  |  |  |
| Q5 |  |  |  |  |  |

RUBRICS FOR CALCULATION:

| Criteria | Weightage | Level 4 – Exemplary | Level 3 – Proficient | Level 2 – Developing | Level 1 – Beginning |
| --- | --- | --- | --- | --- | --- |
| Problem Understanding | 25% | Clearly defines the problem and identifies all relevant constraints accurately | Identifies most constraints with minor gaps | Partial understanding; misses key constraints | Fails to identify constraints correctly |
| Constraint Analysis | 25% | Analyzes constraints effectively and prioritizes them logically | Provides reasonable analysis with some prioritization | Limited analysis; weak prioritization | No meaningful analysis or prioritization |
| Solution Development | 25% | Develops optimal and feasible solutions satisfying all constraints | Develops workable solutions with minor limitations | Solutions partially satisfy constraints | Solutions do not meet constraints |
| Application of Concepts | 15% | Effectively applies appropriate concepts, methods, or tools | Applies concepts with minor errors | Limited application; requires guidance | Unable to apply relevant concepts |
| Justification | 10% | Provides strong justification for decisions with logical reasoning | Provides justification with some clarity | Weak or unclear justification | No justification provided |

CONSTRAINT-BASED PROBLEM SOLVING

QUESTION 1

Construct an I/O communication mechanism for a real-time embedded system with the following constraints:

- CPU utilization must be below 15%

- Multiple sensors generate interrupts every 1 ms

- Use vectored interrupt handling

- Select between memory-mapped and I/O-mapped I/O for optimal latency

ANSWER

1. Introduction

A real-time embedded system must respond to external events within a fixed and predictable time. In this problem, multiple sensors generate interrupts every 1 ms, so the processor has to handle frequent I/O events without spending too much CPU time on communication.

The major constraints are:

- CPU utilization must remain below 15%.

- Multiple sensors generate interrupts every 1 ms.

- Vectored interrupt handling must be used.

- Memory-mapped I/O and I/O-mapped I/O must be compared and the appropriate method selected for optimal latency.

The main objective is to design an I/O communication mechanism that responds quickly while keeping CPU overhead low.

2. Proposed Architecture

The proposed system can be represented as:

Sensors → Interrupt Controller → CPU → Device Registers → Memory Buffer

The sensors generate interrupts whenever new data is available.

The interrupt controller receives these interrupt requests and identifies the source.

Since vectored interrupts are required, every interrupt source can be associated with a specific interrupt vector.

The CPU can directly jump to the appropriate interrupt service routine.

3. Vectored Interrupt Handling

In vectored interrupt handling, each interrupt source has a predefined address or vector.

For example:

Sensor 1 → Vector 1 → ISR 1

Sensor 2 → Vector 2 → ISR 2

Sensor 3 → Vector 3 → ISR 3

This avoids unnecessary software polling of every sensor.

When an interrupt occurs, the processor can directly execute the corresponding ISR.

This reduces interrupt identification time and improves response latency.

4. Sensor Interrupts Every 1 ms

An interrupt every 1 ms means each sensor may generate up to:

1000 interrupts per second

If several sensors are generating interrupts, the total interrupt rate can become very high.

For example, if there are 5 sensors:

5 × 1000 = 5000 interrupts/second

If every interrupt requires a large amount of CPU processing, CPU utilization could exceed the required 15%.

Therefore, the interrupt service routine must be very short.

5. Short Interrupt Service Routine

The ISR should perform only essential operations.

A suitable ISR sequence is:

- Receive interrupt.

- Identify the sensor using the interrupt vector.

- Read or acknowledge the device status.

- Transfer the sensor data to a buffer.

- Clear/acknowledge the interrupt.

- Exit the ISR.

Long processing operations should not be performed inside the ISR.

Instead, the ISR should place data into a buffer and allow a background task to process it.

6. Circular Buffer

A circular buffer can be used to store incoming sensor data.

The structure can be represented as:

Sensor → ISR → Circular Buffer → Processing Task

When a sensor generates an interrupt, the ISR places the new data into the buffer.

The main processing task can later consume the data.

This prevents the CPU from spending too much time inside interrupt routines.

7. Memory-Mapped I/O

In memory-mapped I/O, device registers are assigned addresses within the processor's memory address space.

The CPU accesses the device using normal memory instructions.

For example:

CPU → Memory Address → Sensor Register

Advantages include:

- Simple programming model

- Easy access using normal load/store instructions

- Convenient integration with memory systems

- Suitable for many embedded processors

8. I/O-Mapped I/O

I/O-mapped I/O uses a separate address space for I/O devices.

The processor uses special I/O instructions to communicate with devices.

This separates I/O addresses from normal memory addresses.

9. Selection for Optimal Latency

For this embedded system, memory-mapped I/O is recommended when the processor provides efficient load/store access to device registers.

The reason is that memory-mapped device registers can be accessed directly using normal memory instructions, making the interface simple and efficient.

However, the final latency depends on the processor architecture and bus implementation.

10. Keeping CPU Utilization Below 15%

The most important design decision is to minimize CPU work per interrupt.

The system can achieve this by:

- Using vectored interrupts

- Keeping ISRs short

- Using circular buffers

- Deferring complex processing

- Using efficient register access

- Avoiding polling

For large sensor data transfers, DMA can also be introduced if supported by the system.

11. Proposed Flow

Sensor generates interrupt

↓

Interrupt Controller receives request

↓

Vector identifies sensor

↓

CPU executes short ISR

↓

Sensor data copied to buffer

↓

Interrupt acknowledged

↓

CPU returns to normal task

↓

Background task processes data

This architecture separates real-time interrupt response from data processing.

12. Latency Consideration

The latency consists of:

- Interrupt detection time

- Interrupt controller delay

- Context switching/entry overhead

- ISR execution time

- Device-register access time

Vectored interrupts reduce the software overhead required to identify the source.

Memory-mapped I/O can also provide efficient device-register access.

13. Conclusion

The proposed mechanism uses vectored interrupts, short ISRs, memory-mapped I/O and buffering.

The architecture is:

Sensors → Interrupt Controller → Vectored ISR → Buffer → Background Processing

Memory-mapped I/O is selected for its simple and efficient register access in a suitable embedded architecture.

The design minimizes CPU involvement and helps keep CPU utilization below the required 15%, while still providing fast response to sensor interrupts occurring every 1 ms.

QUESTION 2

Design an I/O subsystem under these constraints:

- High-speed storage requires throughput > 3 GB/s

- Must use NVMe over PCIe

- CPU should not handle bulk data transfer

- Integrate DMA controller with interrupt notification

ANSWER

1. Introduction

High-speed storage systems must transfer large amounts of data quickly without unnecessarily occupying the CPU.

In this problem, the storage subsystem must provide a throughput of more than 3 GB/s and must use NVMe over PCIe.

The CPU should not handle bulk data transfers directly.

Therefore, DMA is an important component of the design.

2. Proposed Architecture

The proposed system is:

CPU → NVMe Driver → PCIe → NVMe Controller → Storage

For data transfer:

NVMe Controller → DMA → Main Memory

After completing the transfer:

DMA/NVMe Controller → Interrupt → CPU

This allows the CPU to initiate an operation without copying every byte of data.

3. NVMe

NVMe stands for Non-Volatile Memory Express.

It is designed for high-speed storage devices, particularly flash-based storage.

NVMe operates over PCIe and supports high parallelism and efficient command processing.

This makes it suitable for high-performance storage requiring throughput above 3 GB/s.

4. PCIe

PCIe provides the high-speed communication link between the processor/system and the NVMe storage controller.

The available throughput depends on PCIe generation, lane count and implementation.

Therefore, the selected PCIe configuration must provide sufficient usable bandwidth to exceed the required 3 GB/s.

5. DMA

DMA stands for Direct Memory Access.

DMA allows a peripheral or controller to transfer data directly between the device and main memory without requiring the CPU to copy every data unit.

For a read operation:

NVMe SSD → DMA → DRAM

For a write operation:

DRAM → DMA → NVMe SSD

The CPU mainly configures the transfer and handles completion rather than performing the bulk movement.

6. Reducing CPU Utilization

Without DMA, the CPU might repeatedly read data from the storage controller and copy it into memory.

For very high-speed storage, this would consume significant CPU resources.

With DMA:

- CPU prepares the command.

- CPU specifies source/destination information.

- DMA/controller performs transfer.

- CPU continues other work.

- Completion interrupt is generated.

This greatly reduces CPU involvement.

7. Interrupt Notification

After the transfer is completed, the NVMe controller can notify the CPU using an interrupt.

The sequence is:

Transfer Started

↓

DMA Transfers Data

↓

Transfer Completed

↓

Interrupt Generated

↓

CPU Executes Completion Handler

This allows the CPU to process the result only when necessary.

8. Queue-Based Operation

NVMe supports command queues.

The system can use queues to maintain multiple outstanding operations.

This improves utilization of the high-speed storage device.

Instead of waiting for one operation to finish before issuing another, multiple commands can be outstanding.

9. Throughput Requirement

The requirement is:

Throughput > 3 GB/s

Therefore, the complete path must support this rate.

The design must consider:

- PCIe bandwidth

- NVMe controller performance

- Storage device speed

- DMA performance

- DRAM bandwidth

- Software overhead

The slowest component can become the bottleneck.

10. Buffering

DRAM can be used as a temporary buffer.

For example:

NVMe SSD → DMA → DRAM Buffer

The application can process the data from the buffer.

Multiple buffers can be used to overlap transfer and processing.

11. Interrupt Coalescing

If the storage subsystem generates an interrupt for every small transfer, CPU overhead can increase.

Interrupt coalescing can reduce this problem by allowing several completed operations to be reported together where appropriate.

This reduces the number of interrupts while maintaining good throughput.

12. Error Handling

The system should also handle:

- DMA errors

- PCIe errors

- Storage errors

- Invalid commands

- Buffer errors

The CPU can handle exceptional situations through interrupts.

13. Overall Data Flow

Application

↓

Operating System / NVMe Driver

↓

NVMe Command Queue

↓

PCIe

↓

NVMe Controller

↓

DMA

↓

DRAM

After completion:

NVMe Controller → Interrupt → CPU

14. Advantages

The proposed architecture provides:

- High throughput

- Low CPU overhead

- Efficient bulk data transfer

- Parallel command processing

- Interrupt-based completion notification

15. Conclusion

The recommended I/O subsystem uses NVMe over PCIe with DMA and interrupt notification.

The CPU initializes and manages operations but does not perform bulk data copying.

The main data path is:

NVMe Storage → PCIe → DMA → DRAM

and completion is reported using an interrupt.

By selecting a PCIe configuration with sufficient bandwidth and combining it with NVMe queues, DMA and efficient buffering, the system can be designed to achieve the required greater than 3 GB/s throughput while keeping CPU overhead low.

QUESTION 3

Construct a multi-device communication architecture with constraints:

- 6 peripherals using USB, 2 high-speed devices using Thunderbolt

- Avoid interrupt starvation

- Use hardware and software interrupts

- Ensure fair bus arbitration

ANSWER

1. Introduction

A computer system may need to communicate with many peripherals simultaneously.

In this problem, the system contains:

- 6 USB peripherals

- 2 high-speed Thunderbolt devices

The architecture must avoid interrupt starvation, use both hardware and software interrupts, and provide fair bus arbitration.

The main challenge is that high-speed devices can generate large amounts of traffic and potentially prevent slower devices from receiving sufficient bus service.

2. Proposed Architecture

The architecture can be represented as:

CPU

↓

Interrupt Controller

↓

I/O Controller

↙ ↓ ↘

USB Controller | Thunderbolt Controller | Other I/O

The USB controller manages six USB peripherals.

The Thunderbolt controller manages two high-speed devices.

3. USB Devices

The six USB peripherals can include devices such as:

- Keyboard

- Mouse

- Sensor

- Printer

- Camera

- Storage device

They communicate through the USB controller.

The controller manages communication and presents events to the processor.

4. Thunderbolt Devices

Thunderbolt devices provide high-speed communication.

Because two Thunderbolt devices are operating alongside six USB peripherals, bus arbitration must ensure that high-bandwidth traffic does not completely dominate the system.

5. Interrupt Types

Two types of interrupts are required:

Hardware Interrupts

Hardware interrupts originate from physical devices.

For example:

USB Device → Hardware Interrupt

or:

Thunderbolt Controller → Hardware Interrupt

These are appropriate for urgent events requiring immediate processor attention.

Software Interrupts

Software interrupts are generated by software.

They can be used for:

- Background processing

- Deferred work

- Communication between software components

- Lower-priority tasks

6. Avoiding Interrupt Starvation

Interrupt starvation occurs when lower-priority devices continuously wait because higher-priority interrupts are always being serviced.

This can be avoided by:

- Priority management

- Interrupt masking where appropriate

- Interrupt moderation

- Fair scheduling

- Deferred processing

High-priority events should be serviced quickly, but lower-priority devices should still receive CPU time.

7. Fair Bus Arbitration

Bus arbitration determines which device gets access to a shared communication resource.

A fair system can use round-robin arbitration.

For example:

Device 1 → Device 2 → Device 3 → Device 4 → ...

After one device receives service, the next eligible device gets its opportunity.

This prevents a single high-speed device from continuously occupying the bus.

8. Weighted Arbitration

Because Thunderbolt devices may require more bandwidth than simple USB peripherals, the system can use controlled weighted scheduling.

For example:

- Thunderbolt devices receive sufficient bandwidth for high-speed traffic.

- USB devices receive guaranteed service intervals.

The exact weights depend on workload requirements.

9. Interrupt Priority

A practical priority arrangement can be:

Emergency/time-critical hardware interrupt

↓

High-speed device interrupt

↓

Normal USB hardware interrupt

↓

Software/background interrupt

However, priorities should not permanently block lower-level interrupts.

10. Interrupt Coalescing

For high-speed devices, generating one interrupt for every small event can overload the CPU.

Interrupt coalescing can combine multiple events.

For example:

Multiple Thunderbolt events → One interrupt

This reduces interrupt overhead.

11. Software Interrupts for Deferred Processing

The hardware ISR should perform only the minimum required operations.

After receiving an event, it can schedule software-level processing.

For example:

Hardware Interrupt → Short ISR → Software Interrupt/Deferred Task → Data Processing

This keeps hardware interrupt latency low.

12. Buffering

Each device can have its own buffer.

For example:

USB 1 → Buffer 1

USB 2 → Buffer 2

...

Thunderbolt 1 → High-speed Buffer

Thunderbolt 2 → High-speed Buffer

Buffers prevent temporary traffic bursts from immediately blocking other devices.

13. Fairness

Fair arbitration ensures that:

- All USB peripherals receive service.

- Thunderbolt devices receive sufficient high-speed bandwidth.

- No device monopolizes the bus.

- Lower-priority events eventually get processed.

14. Proposed Flow

USB/Thunderbolt Device

↓

Controller

↓

Interrupt Controller

↓

Priority/Fairness Logic

↓

CPU ISR

↓

Buffer

↓

Software Processing

15. Conclusion

The proposed architecture uses separate USB and Thunderbolt controllers connected through an interrupt and bus-management system.

Hardware interrupts are used for real device events, while software interrupts are used for deferred/background processing.

Round-robin or controlled weighted arbitration can provide fairness.

Interrupt coalescing and buffering can reduce CPU overhead.

Therefore, the architecture can support 6 USB peripherals and 2 Thunderbolt devices while avoiding interrupt starvation and ensuring fair access to system resources.

QUESTION 4

Design a data acquisition system with constraints:

- Continuous data stream at 500 MB/s

- Minimal CPU intervention required

- Must compare DMA transfer vs interrupt-driven I/O

- Use memory-mapped I/O for device registers

ANSWER

1. Introduction

A data acquisition system collects continuous data from a device or sensor and transfers it into memory for processing.

In this problem, the system must handle a continuous data stream of 500 MB/s.

This is a relatively high data rate, so the design must minimize CPU intervention.

The system must also compare DMA transfer and interrupt-driven I/O and must use memory-mapped I/O for device registers.

2. Proposed Architecture

The recommended architecture is:

Data Acquisition Device

↓

Device Registers

↓

DMA Controller

↓

DRAM Buffer

↓

Processing Application

The CPU configures the device and DMA controller but does not transfer every byte itself.

3. Memory-Mapped I/O

The device registers are assigned addresses in the processor's memory address space.

The CPU can access them using normal load/store operations.

For example:

CPU → Memory Address → Device Register

Registers may contain:

- Control information

- Status

- Data configuration

- DMA configuration

- Interrupt status

4. DMA Transfer

DMA allows data to move directly between the data acquisition device and memory.

The process is:

- CPU configures DMA.

- Device begins producing data.

- DMA transfers data to memory.

- DMA updates transfer information.

- Completion or threshold interrupt is generated.

- CPU processes the buffer.

This greatly reduces CPU intervention.

5. Interrupt-Driven I/O

In interrupt-driven I/O, the device generates an interrupt when it needs CPU attention.

The CPU executes an ISR and transfers or processes the data.

For a low data rate, this can be acceptable.

However, at 500 MB/s, interrupt-driven transfer can generate a very large number of interrupts.

This can consume substantial CPU time.

6. DMA vs Interrupt-Driven I/O

| Feature | DMA | Interrupt-Driven I/O |
| --- | --- | --- |
| CPU involvement | Low | High |
| Bulk transfer | Excellent | Less suitable |
| Interrupt frequency | Lower | Potentially high |
| Efficiency at 500 MB/s | High | Lower |
| Best use | Continuous high-speed data | Small/occasional transfers |

7. Why DMA Is Preferred

The requirement explicitly states that minimal CPU intervention is required.

Therefore, DMA is the better choice.

Instead of:

Device → CPU → Memory

the system uses:

Device → DMA → Memory

The CPU is informed only when meaningful events occur.

8. Double Buffering

Double buffering can improve continuous data acquisition.

Two buffers can be used:

Buffer A → Processing

while:

Buffer B ← DMA

When Buffer B becomes full, DMA switches to Buffer A.

This allows acquisition and processing to overlap.

9. Circular Buffer

A circular buffer can also be used for continuous streaming.

The DMA controller continuously writes incoming data into available portions of the buffer.

The software reads processed portions.

This is useful when the stream does not stop.

10. Interrupt Strategy

The system should not generate an interrupt for every byte or very small transfer.

Instead, an interrupt can occur when:

- A buffer becomes full

- A defined threshold is reached

- A DMA block is completed

- An error occurs

This dramatically reduces CPU interrupt overhead.

11. Throughput Requirement

The system must continuously sustain:

500 MB/s

Therefore, all components must support this rate.

The design should consider:

- Device output rate

- DMA bandwidth

- DRAM bandwidth

- Bus bandwidth

- Buffer size

- Processing speed

12. CPU Utilization

With interrupt-driven I/O, the CPU may repeatedly execute:

Interrupt → ISR → Data Transfer → Return

At a high data rate, this becomes inefficient.

With DMA:

CPU → Configure DMA → Continue other work

and later:

DMA → Completion Interrupt → CPU

This is significantly more efficient.

13. Error Handling

The DMA controller and acquisition device should detect:

- Buffer overflow

- Transfer failure

- Device errors

- Invalid memory address

- Data loss

Error interrupts can notify the CPU.

14. Final Architecture

Data Acquisition Device

↓

Memory-Mapped Control Registers

↓

DMA Controller

↓

Circular/Double Buffer in DRAM

↓

Processing Task

↓

Output/Application

15. Conclusion

For a continuous 500 MB/s data stream, DMA is clearly more suitable than interrupt-driven I/O because it minimizes CPU intervention.

Interrupt-driven I/O is useful for control events and smaller transfers, but using it for every data movement at 500 MB/s would create excessive CPU overhead.

Therefore, the recommended design uses:

Memory-Mapped I/O + DMA + Double/Circular Buffer + Completion Interrupts

This provides efficient continuous data acquisition while keeping CPU involvement minimal.

QUESTION 5

Construct an interrupt handling mechanism with constraints:

- Support nested interrupts

- Maximum interrupt latency < 10 μs

- Use vectored interrupts for high-priority devices

- Use software interrupts for background tasks

ANSWER

1. Introduction

Interrupts allow hardware and software events to receive processor attention without continuous polling.

In a real-time system, interrupt handling must be carefully designed because excessive interrupt latency can cause important events to be missed or processed too late.

The given system has four major requirements:

- Support nested interrupts.

- Maximum interrupt latency must be less than 10 μs.

- Vectored interrupts must be used for high-priority devices.

- Software interrupts must be used for background tasks.

2. Proposed Architecture

The architecture is:

Device

↓

Interrupt Controller

↓

Priority Logic

↓

Vector Table

↓

High-Priority ISR

↓

Deferred Software Task

The interrupt controller determines which event requires processor attention.

3. Nested Interrupts

Nested interrupts allow a high-priority interrupt to interrupt a currently executing lower-priority ISR.

For example:

Low-Priority ISR running

↓

High-Priority Interrupt arrives

↓

Current ISR state saved

↓

High-Priority ISR executes

↓

Return to Low-Priority ISR

This ensures that critical events do not have to wait for lengthy lower-priority processing.

4. Interrupt Priority

Each interrupt source should have a priority.

For example:

Priority 1 → Critical sensor

Priority 2 → Communication device

Priority 3 → Normal peripheral

Priority 4 → Background software task

A higher-priority interrupt can preempt a lower-priority ISR.

5. Vectored Interrupts

High-priority devices should use vectored interrupts.

Each device is associated with a vector.

For example:

Device A → Vector A → ISR A

Device B → Vector B → ISR B

This avoids software polling and reduces interrupt identification time.

6. Maximum Latency < 10 μs

Interrupt latency is the time between an interrupt request and the beginning of its service routine.

The design must ensure:

Maximum interrupt latency < 10 μs

Several factors affect latency:

- Interrupt controller delay

- CPU state saving

- Priority determination

- Interrupt masking

- Cache behavior

- ISR entry overhead

Therefore, the architecture must minimize unnecessary work before entering the high-priority ISR.

7. Short High-Priority ISR

The high-priority ISR should be extremely short.

It should:

- Save required state.

- Read critical device status.

- Capture necessary data.

- Acknowledge the interrupt.

- Schedule additional processing.

- Return as soon as possible.

Complex calculations should not be performed inside the high-priority ISR.

8. Software Interrupts

Software interrupts can be used for background tasks.

For example:

Hardware Interrupt → Minimal ISR → Software Interrupt → Background Processing

This keeps the hardware ISR short.

Software interrupts are useful for deferred processing that does not need the same immediate priority as the original hardware event.

9. Nested Interrupt Operation

Suppose a low-priority interrupt is currently executing.

A high-priority interrupt occurs.

The system performs:

Low ISR

↓

High-priority interrupt

↓

Save low ISR state

↓

Execute high-priority ISR

↓

Restore low ISR state

↓

Continue low ISR

This provides rapid response to critical events.

10. Interrupt Masking

Interrupt masking must be used carefully.

If interrupts remain disabled for too long, high-priority events may experience excessive latency.

Therefore, the system should disable interrupts only for very short critical sections.

High-priority interrupts should remain enabled whenever safe.

11. Priority-Based Interrupt Controller

A programmable interrupt controller can manage priorities.

It can:

- Detect interrupt requests

- Determine priority

- Select the highest-priority pending interrupt

- Provide the vector

- Support nested interrupts

12. Background Processing

Background tasks should not be placed inside critical ISRs.

For example:

High-Priority Sensor Interrupt

→ Capture sensor value

→ Acknowledge interrupt

→ Schedule software task

Then:

Software Interrupt

→ Perform detailed data processing

This approach reduces interrupt latency.

13. Example Timing Strategy

To satisfy the <10 μs requirement:

- Interrupt recognition should be fast.

- Vector lookup should be direct.

- High-priority ISR should be short.

- Long operations should be deferred.

- Interrupt masking should be minimized.

- Nested interrupt support should be enabled.

The exact numerical timing must be validated on the target processor and hardware implementation.

14. Advantages

The proposed mechanism provides:

- Fast response

- Support for critical devices

- Nested interrupt capability

- Low interrupt latency

- Reduced ISR execution time

- Efficient background processing

15. Overall Flow

High-Priority Device

↓

Interrupt Request

↓

Interrupt Controller

↓

Priority Check

↓

Vector Lookup

↓

High-Priority ISR

↓

Acknowledge + Capture Data

↓

Software Interrupt

↓

Background Processing

If another higher-priority event arrives during a lower-priority ISR:

Lower ISR → Preempted → Higher ISR → Resume Lower ISR

16. Conclusion

The required interrupt mechanism should combine nested interrupts, vectored interrupt handling, short high-priority ISRs and software interrupts.

High-priority devices receive vectored interrupts so that the CPU can directly reach the correct ISR.

Nested interrupts allow critical events to preempt lower-priority processing.

Software interrupts are used for background and deferred tasks.

To achieve the required maximum interrupt latency below 10 μs, the system must minimize interrupt-entry overhead, keep high-priority ISRs short and avoid long periods of interrupt masking.

The final mechanism is:

Priority Device → Vectored Hardware Interrupt → Short ISR → Deferred Software Interrupt → Background Processing

This provides fast real-time response while maintaining efficient CPU utilization.